Z-Wave Open Sensor

Guide Document

1. Open Sensor  Z-Wave Specifications

- Device Type  :  Sensor – Notification

- Role Type    :  RSS (Reporting Sleeping Slave)

- Command Class Support/Control

- Version Command Class version 2

- Manufacture Specific Command Class Version 2

- Notification Command Class Version 3

- Device Reset Locally Command Class Version 1

- Association Group Information Command Class Version 1

- Association Command Class Version 2

- Battery Command Class Version 1

- Wake Up Command Class Version 2

- Power Level Command Class Version 1

- Z-Wave plus info Command Class Version 2

- Firmware Update MD Command Class Version 2

- Configuration Command Class Version 1

2. Technical Specifications

- South Korea 919-923 MHz RF Frequency

- Up to 100m outdoor line of sight

- Used AAA type Battery.

- Operating Temperature 0 oC ~ 50 oC.

3. Open Sensor Device Inclusion

- Press & hold Inclusion/Exclusion button for 5 seconds when Open Sensor is exclusion status.

- LED of Open sensor will be on 5 seconds later.

- Open sensor will try Inclusion for 30sec when button is released.

- LED will blink 3 times with 250ms interval if Inclusion is successful.

4. Open Sensor Device Exclusion

- Receiver will stay exclusion waiting mode.

- Press & hold Inclusion/Exclusion button for 5 seconds when Open Sensor is Inclusion status.

- LED of Open sensor will be on 5 seconds later.

- Open sensor will try Exclusion when button is released.

- LED will blink 3 times with 250ms interval if Exclusion is successful.

5. Open Sensor Device Factory Reset

- Press & hold Inclusion/Exclusion button for 10sec.

- LED will blink while holding the button.

- Factory reset will be done in 5sec after releasing button and LED will blink 5 times with 100ms interval.

Please use this procedure only when the network primary controller is missing or otherwise inoperable.
6. Notification Command

| No | Description | Byte size | Value |  |
| --- | --- | --- | --- | --- |
| 1 | Notification Type | 1 | 0x06 | Access Control |
| 2 | Event | 1 | 0x16/0x17 | Open/Close |

7. Wake up Notification

- Device sends Wake up Notification by every 15 minutes.

- Devices also sends Wake up Notification when inclusion/exclusion button is shortly pressed regardless of 15minutes interval.

- Open Event : about 15~20 mm.

- Close Event : about 5~7 mm.

8. Association& Association Group Information

- Grouping identifier : Association Group support is only 1.

- Maximum number of devices that can be added to the group : The maximum number of node supports up to 5.

- Description of how the association group is used and/or triggered by the product  : This product's Association Group are used by Life Line of products.

9. Battery Report

- The device sends Battery report when the power supplied

- The device sends Battery Report 4times a day for every 6 hours. (Default)

- The device sends Low Battery Report 1 time if battery voltages drops under 2.5V

| Battery Voltage | % | Data |
| --- | --- | --- |
| 3.2V | 100% | 64(h) |
| ∙∙∙ | ∙∙∙ | ∙∙∙ |
| 2.5V(Low battery warning) | 10% | FF(h) |
| ∙∙∙ | ∙∙∙ | ∙∙∙ |
| 2.3V | 0% | 00(h) |

10. Z-Wave Configuration Parameters

- Parameter Number : Battery Period (0x01).

- Description of parameter and its effect on the product

: Battery Report Period Value

- Default value : 0x18

- Size (Number of bytes) : 1 Byte

- Possible Values : 0x01 ~ 0xFF

11. ZWAVE_PLUS_ INFO

| No | Description | Byte size | Value |  |
| --- | --- | --- | --- | --- |
| 1 | Z-Wave+ Version | 1 | 0x01 |  |
| 2 | Role Type | 1 | 0x06 | RSS |
| 3 | Node Type | 1 | 0x00 | Z-Wave Plus node |
| 4 | Installer Icon Type | 2 | 0x0C06 | ICON_TYPE_SPECIFIC_ SENSOR_NOTIFICATION_ ACCESS_CONTROL |
| 5 | User Icon Type | 2 | 0x0C06 |  |

12. Version Command

| No | Description | Byte size | Value |  |
| --- | --- | --- | --- | --- |
| 1 | Z-Wave Library Type | 1 | 0x03 | Enhanced Slave |
| 2 | Z-Wave Protocol Version | 2 | 0x0405 |  |
| 3 | Firmware Version | 2 | 0x2714 | v1.00.04 |
| 4 | Hardware Version | 1 | 0x03 | V1.3 |

13. Manufacturer Command

| No | Description | Byte size | Value | 비고 |
| --- | --- | --- | --- | --- |
| 1 | Manufacturer ID | 2 | 0x022C | RemoteSolution |
| 2 | Product Type ID | 2 | 0x0003 | ZWAVE PLUS |
| 3 | Product ID | 2 | 0x0001 | Sensor Door |
| 4 | Device ID Type | 3bits | 1 | Serial Number |
| 5 | Device ID Data Format | 3bits | 1 | Binary |
| 6 | Device ID Data Length Indicator | 5bits | 6 | Device ID Data Length Indicator |
| 7 | Device ID Data 1 ~ 6 | 6 | 022C44000001 | Serial Number |

14. Devices from multiple manufacturers in one network

This product can be included and operated in any Z-Wave network with other Z-Wave certified devices from other manufacturers and/or other applications.

All non-battery operated nodes within the network will act as repeaters regardless of vendor to increase reliability of the network.

15. Support of Basic CC is not required for this Device Type.

So ignore if you get Basic Get.