B400-ZW Alliance Manual

Dawon DNS Co., LTD

Date : 2015. 05. 26

This is a device that have standby power cut-off function, electricity consumption measurements and z-wave communication.

D.1 Slave Device : Inclusion/Exclusion

Inclusion/Exclusion : In the state Smart Plug is plugged into the power outlet, you press the button of Smart Plug with your finger to 5 seconds continuously  and if you take your finger off when the red LED flashes the device is in the Inclusion or Exclusion.

(Smart Plug가 전원 콘센트에 꽂혀있는 상태에서 Smart Plug의 Button을 손가락으로 5초간 지속적으로 눌러서 적색 LED가 점멸하게 될 때 손가락을 떼면 Inclusion 또는 Exclusion을 하게 되어 있습니다.)

- Inclusion : After mounting the UZB controller to your PC and run the PC Controller program you can click the Add button in the PC Controller program. Press the button on the Smart Plug for more than five seconds after you plug the Smart Plug into a power outlet will Inclusion in the network controller, while the flashing red button LED Smart Plug.

(UZB콘트롤러를 PC에 부착하고 PC Controller 프로그램을 실행한 후 PC Controller 프로그램의 Add버튼을 누릅니다. Smart Plug를 전원 콘센트에 꽂은 후 Smart Plug의 버튼을 5초이상 누르면 Smart Plug의 버튼 LED가 적색으로 점멸하면서 콘트롤러 네트워크에 Inclusion이 됩니다.)

- Exclusion : After mounting the UZB controller to your PC and run the PC Controller program you can click the Remove button in the PC Controller program. Press the button on the Smart Plug for more than five seconds after you plug the Smart Plug into a power outlet will Exclusion in the network controller, while the flashing red button LED Smart Plug.

(UZB콘트롤러를 PC에 부착하고 PC Controller 프로그램을 실행한 후 PC Controller 프로그램의 Remove버튼을 누릅니다. Smart Plug를 전원 콘센트에 꽂은 후 Smart Plug의 버튼을 5초이상 누르면 Smart Plug의 버튼 LED가 적색으로 점멸하면서 콘트롤러 네트워크에서 Exclusion이 됩니다.)

- Factory Reset : Press the button on the Smart Plug for more than 10 seconds after you plug the Smart Plug into a power outlet will Exclusion in the network controller, while the flashing red button LED Smart Plug. And all the variables are initialized.

※ Please use this procedure only in the event that the network primary controller is missing or otherwise inoperable

(Smart Plug를 전원 콘센트에 꽂은 후 Smart Plug의 버튼을 10초이상 누르면 Smart Plug의 버튼 LED가 적색으로 점멸하면서 콘트롤러 네트워크에서 Exclusion이 됩니다. 그리고 모든 변수는 초기화 됩니다.)

Button of Smart Plug

D.7 Type

1) ROLE TYPE : AOS(Always On Slave) - ZWAVEPLUS_INFO_REPORT_ROLE_TYPE_SLAVE_ALWAYS_ON

2) DEVICE TYPE : Switch On/Off

- Generic : GENERIC_TYPE_SWITCH_BINARY

- Specific : SPECIFIC_TYPE_POWER_SWITCH_BINARY

3) NODE TYPE : Z-Wave plus

- ZWAVEPLUS_INFO_REPORT_NODE_TYPE_ZWAVEPLUS_NODE

- Node Info List

COMMAND_CLASS_ZWAVEPLUS_INFO

COMMAND_CLASS_VERSION

COMMAND_CLASS_MANUFACTURER_SPECIFIC

COMMAND_CLASS_DEVICE_RESET_LOCALLY

COMMAND_CLASS_ASSOCIATION

COMMAND_CLASS_ASSOCIATION_GRP_INFO

COMMAND_CLASS_POWERLEVEL

COMMAND_CLASS_SWITCH_BINARY

COMMAND_CLASS_METER_V3

COMMAND_CLASS_CONFIGURATION

COMMAND_CLASS_FIRMWARE_UPDATE_MD_V2

D.9 Protocol (This Device is also compatible with other well-Device.)

This product can be included and operated in any Z-Wave network with other Z-Wave certified devices from other manufacturers and/or other applications. All non-battery operated nodes within the network will act as repeaters regardless of vendor to increase reliability of the network.

| No | Command Class | Command Name | Value |  | Operation |
| --- | --- | --- | --- | --- | --- |
| 1 | COMMAND_CLASS_BASIC_V1 | BASIC_GET | BASIC_REPORT |  | Relay Status |
|  |  | BASIC_SET | 0xFF |  | Relay ON |
|  |  |  | 0x00 |  | Relay OFF |
| 2 | COMMAND_CLASS_SWITCH_BINARY_V1 | SWITCH_BINARY_SET | 0xFF |  | Relay ON |
|  |  |  | 0x00 |  | Relay OFF |
|  |  | SWITCH_BINARY_GET | SWITCH_BINARY_REPORT |  | Relay Status Value |
| 3 | COMMAND_CLASS_METER_V3 | When the device requests a METER_GET reports the value set in the Scale. | METER_REPORT |  |  |
|  |  |  | Rate Type: METER_REPORT_RATE_TYPE_IMPORT_V4(0x01) Meter Type : METER_REPORT_METER_TYPE_ELECTRIC_METER_V3(0x01) Delta Time : Transmission Period(in minutes), Previous Meter Value : Previous Value |  |  |
|  |  |  | Scale:0x00 | Report the KWh(4 byte) value |  |
|  |  |  | Scale:0x02 | Report the Watt(4byte) value The hex data of device must be divided by 100 because this data is 1/100 Watt unit. |  |
|  |  | METER_RESET |  | initialize the KWh value |  |
|  |  | METER_SUPPORTED_GET |  |  |  |
|  |  | METER_SUPPORTED_REPORT | Scale: 0 or 2 | Meter Type:0x01, Scale Supported:0x05 |  |
| 4 | COMMAND_CLASS_CONFIGURATION_V1 | CONFIGURATION_SET | Parameter Number : 1 Size : 4 | (Standby Power Setting Value : 3 Byte, Standby Power Enable/Disble : 1 Byte) Value : if the value is set by 300, standby power equals 3.00w (0x00012C) and standby power enable is 1. total 0x00012C01 ( Default : 0 – 0.00W, StandbyPower Disable) |  |
|  |  |  | Parameter Number : 2 Size : 1 (This configuration value is used only 0 and 1.) | Value : 1 Periodic Measurement Value Transmission Enable Value : 0 Periodic Measurement Value Transmission disable ( Default : Periodic Measurement Value Transmission Enable ) |  |
|  |  |  | Parameter Number : 3 Size : 1 (This configuration value is used only 0 and 1.) | Value : 0 Accumulation Stop Value : 1 Accumulation Start ( Default : Accumulation Start ) |  |
|  |  |  | Parameter Number : 4 Size : 1 (This configuration value is used only 0 and 1.) | Value : 0 Connected Device Not Use Value : 1 Connected Device Use - Before setting the standby power it does not report. |  |
|  |  |  | Parameter Number : 5 Size : 1 | Minimum time interval : 10minutes Value : 1 (10minutes * 1) = 10minutes interval Value : 2 (10minutes * 2) = 20minutes interval …….. …….. Value : 144(10m * 144) = 24 hours interval =>MAX ( Default : 6 = One hour ) |  |
|  |  | CONFIGURATION_GET | Parameter Number: 1~5 | CONFIGURATION_REPORT |  |
| 5 | COMMAND_CLASS_NOTIFICATION | NOTIFICATION_REPORT | V1 Alarm Type : 0x00 | None Supported |  |
|  |  |  | Notification Status | 0xFF : Unsolicited notification |  |
|  |  |  | Notification Type : 0x08 | Power Management Event - 0x02 : Relay OFF - 0x03 : Relay ON - 0x06 : OverCurrent |  |
|  |  |  | Notification Type : 0x09 | Notification Event System - 0x01 : Hardware failure(Internal Operation failure) |  |
| 6 | COMMAND_CLASS_VERSION | VERSION_GET |  |  |  |
|  |  | VERSION_REPORT | Library Type | 1byte |  |
|  |  |  | Protocol Version | 2byte |  |
|  |  |  | Application Version | 2byte – When the application firmware is update, this value is changed. (Start at 1.00) |  |
| 7 | Unsolicited Report | NOTIFICATION_REPORT | Notification Type : 0x08 Relay ON : 0x03 Relay OFF : 0x02 | When unplugging from the wall outlet and plugged into a smart plug. The device is based on the current state relays Report to Notification. Or The Report also states that the relay current value of the device changed. |  |
|  |  |  | Notification Type : 0x08 OverCurrent : 0x06 | When over-current occurs, the device shuts off the relay. The device Report the over-current. |  |
|  |  |  | Notification Type : 0x09 Hardware Fail : 0x01 | Device must report when it is not the normal operation as an internal malfunction. |  |
|  |  | MeterV3_REPORT | Rate Type: METER_REPORT_RATE_TYPE_IMPORT_V4(0x01) Meter Type : METER_REPORT_METER_TYPE_ELECTRIC_METER_V3(0x01) Delta Time : Transmission Period(min unit), Previous Meter Value : Previous Value |  |  |
|  |  |  | Scale:0x00 | Report the KWh(4byte) value. - In addition to reporting regularly even when plugged in to a power source and the first report at a time. |  |
|  |  | CONFIGURATION_REPORT | Parameter Number : 5 Size : 1 | When connected equipment is changed to Use in the Not Use, when the device is changed to Not Use In Use. |  |
|  |  |  |  | The standby power of connected devices to become this feature is enabled must be set. Or used the Default value. |  |
|  |  | VERSION_REPORT | Library Type | 1byte |  |
|  |  |  | Protocol Version | 2byte |  |
|  |  |  | Application Version | 2byte – If the Application firmware is being upgraded this value is changed. (Starting at 1.00) If the firmware is upgraded when you turn off the main power, the plug must report the version . |  |

D.10 Key Features

1) Inclusion/Exclusion by button (Press the button for 5 seconds until the LED blinking)

2) Relay On/Off manually (Press the button shortly)

3) Basic Set Commnad : Relay On – Value : 0xff, Relay Off – Value : 0x00

Ack : Notification – 0x03 : RelayOn, 0x02 : RelayOff

4) LED color : Relay On – Green Color, Relay Off – RED Color

5) Meter V3 Command – 0x00 : Wh(Energy), 0x02 : Watt(Power)

6) Grouping identifier : Association Group support is only 1.

7) Maximum number of devices that can be added to the group : The maximum number of node supports up to 5.

8) Description of how the association group is used and/or triggered by the product  : This product's Association Group are used by Life Line of products.