







 HS1CA-Z
Z-Wave Device

Guide Document
  [image: image1.png]



1-Device Preheat
Power the device on, a “Di” sound, then green LED, red LED, yellow LED flash alternately about 90 sec, enter into normal model, after that, the device will be able to be included into network.  
2-Device Inclusion

-  Click [Add] icon in Z-Wave PC Controller Program.
-  Press the Net_Button 3 times within 1.5s, Green LED is Blinking 3 times within 1 second.
-  If Inclusion Process is successful, Green LED will turn off.

3-Device Exclusion

-  Click [Remove] icon in Z-Wave PC Controller Program

-  Press the Net_Button 3 times within 1.5s 

-  If Exclusion Process is successful, Green LED is Blinking 6 times, then turn off.
4-Device test mode 
Press Test_Button to test if the device is working normally, green LED, red LED, yellow LED and green LED flash alternately once, and “Di-Di-Di-Di” sound when CO is detected.

5-Device detection mode 
if Inclusion process is successful, the device will report alarm information if detect the status change of CO and no CO . Red LED flash 4 times every 10 seconds, and “Di-Di-Di-Di” sound when CO is detected. 
6-Device Factory Reset

-Long press Net_Button while installing batteries in HS1CA-Z. 

- Device Reset Locally notification is Transmitted.

- Please use this procedure only when the network primary controller is 

missing or otherwise inoperable.

7-Association & Association Group Information

-  Lifeline between controller and HS1CA-Z
- Supported command classes

Battery report, Notification report, and Device Reset Locally notification, Binary report.
- Maximum supported nodes are 5.

- Association Group description
Association group 1: Lifeline association group
Include command classes: Battery report, Notification report, and Device Reset Locally notification, Binary report.

▶ Association group 2: Root Device group(Binary Sensor)

Binary Sensor Command Class: Compatible with 300 series

Binary Sensor reports status of CO or no CO via Lifeline.

2-When the sensor detects status change of CO and no CO, the device will be triggered. 

▶Association group 3: Root Device group(Notification)

Notification reports status of detect CO or no CO via Lifeline.

When the sensor detects status change of CO and no CO, the device will be triggered.  

8-Notification Report

-Press or release test_button, or CO status is changed(CO detected or no CO detected) , Notification Report is transmitted

	Notification Type 
	Event

	CO Alarm

(0x02)
	Previous Events cleared ( 0x00)

	
	Carbon monoxide detected,
Unknown Location(0x02)


9-Battery Report
-  Battery Report is transmitted when power on the device.
-  Battery Report is transmitted once a day.
-  Battery Report is transmitted when the battery falls less than 10%
-

	Battery Voltage
	Battery Level

	2.6V ~ 3.0V
	10~ 100 (%)

	Under 2.6V  
	0xFF


10-Wake up Notification

-  Wake up Notification is transmitted every 24 hours by default.

-  Wake up Notification is transmitted after Notification Report is Transmitted
11-Devices from multiple manufacturers in one network

-  This product can be included and operated in any Z-Wave network with

other Z-Wave certified devices from other manufacturers and/or other

applicants. All non-battery operated nodes within the network will act as

repeaters regardless of vendor to increase reliability of the network.
12-Z-Wave Specifications

 ■Device Type : Sensor - Notification

■Role Type : RSS (Reporting Sleeping Slave)

■Command Class Support/Control

 ▶Z-Wave plus info

 ▶Version

 ▶Manufacture Specific

 ▶Notification

 ▶Device Reset Locally

 ▶Association Group Information
▶Association

 ▶Battery

 ▶Wake Up

 ▶Power Level

▶Binary Sensor
13-Indicate LED statues description-

	status
	LED status
	Alarm sound

	Preheat （预热）
	Green LED, red LED, yellow LED flash alternately
	        No sound

	Normal 
	Green LED flash 1 time every 53 seconds
	        No sound

	Test mode
	Green LED, red LED, yellow LED and green LED flash alternately
	“Di-Di-Di-Di”sound loudly

	Alarm
	Red LED flash 4 times every 10 seconds
	“Di-Di-Di-Di”sound loudly

	Low battery
	Yellow LED flash 1 time every 53 seconds
	 “Di” sound every 53 seconds

	Fault 
	Yellow LED flash 2 times every 10 seconds
	“Di” sound twice every 10 seconds


14-How to replace the batteries
	[image: image2.png]<





	Turn right to dismantle the mounting base of the device

	[image: image3.png]<GFF TWIST ONZ





	1)take out battery from the device

2)Insert the battery into the device
( You must be careful of the battery

polarity)

	[image: image4.png]



	Turn left to install the mounting base of the device

(before install the mounting base, you should must insert battery firstly , otherwise the mounting base will not be fixed)


15-Technical Specifications
	Model No.
	HS1CA-Z

	RF frequency
	868.40- 869.85 MHz 

	RF operating distance
	Up to 100m outdoor line of sight

Up to 40m indoor

	Powered by
	1 x CR123A 

	Temperature
	-10℃~ 50℃

	Humidity
	95% max

	Weight
	66g (with batteries)


Net_Button





Test_Button/LED Indicator









