







 HS1CG-Z
Z-Wave Device

Guide Document
[image: image1.png]


  
  [image: image2.png]



1-Device pre-heat

Power the device on, a “Di” sound, then green LED, red LED flash alternately about 180 sec, enter into normal model and Green LED is always on , after that, the device will be able to be included into network.
2-Device Add
- Click [Add] icon in Z-Wave Controller.
- Press the Net_Button 3 times within 1.5s, Green LED is Blinking 3 times within 1 second.
- If add Process is successful, Green LED will turn on.

3-Device Remove 
- Click [Remove] icon in Z-Wave Controller. 
- Press the Net_Button 3 times within 1.5s 

- If remove Process is successful, Green LED is Blinking 6 times, then turn on
4-Device Factory Reset

-Long press Net_Button at least 10 seconds in the device. 

-Device Reset Locally notification is Transmitted.

-Please use this procedure only when the network primary controller is missing or otherwise inoperable.
5-Association & Association Group Information

- Lifeline between controller and the device
- Supported command classes:
Notification report, and Device Reset Locally notification.
- Association Group description:

Association group 1: Lifeline association group
- Maximum supported node is 1.

Include command classes: Notification Report and Device Reset Locally notification.

▶Association group 2: Root Device group(Notification)

Maximum supported nodes are 5.
Notification reports status of gas detected or no gas detected via Lifeline.

2-When the sensor detects status change of gas detected and no gas detected, the device will be triggered.
6- Notification Types and Events 
 -Press or release Test_Button, or Gas status is changed(Gas detected or no Gas detected) , Notification Report is transmitted

Notification TYEP
	      Notification Type
	Event

	GAS Alarm(0x12)
	Previous Events cleared ( 0x00)
(V7)

	
	Combustible Gas detected Unknown Location(V7)


7-Devices from multiple manufacturers in one network

-  This product can be included and operated in any Z-Wave network with

other Z-Wave certified devices from other manufacturers and/or other

applicants. All non-battery operated nodes within the network will act as

repeaters regardless of vendor to increase reliability of the network.
8-Z-Wave Specifications

 ■Device Type : Sensor - Notification

■Role Type :Always On Slave

■Command Class Support/Control

 ▶Z-Wave plus info

 ▶Version

 ▶Manufacture Specific

 ▶Device Reset Locally

 ▶Association Group Information
▶Association

 ▶Power Level

▶Notification

9-Device LED status and Test mode

-LED status
	Item
	Color
	LED status
	sound

	Power
	Green
	Always
	NO

	Alarm
	RED
	Blink
	“Di-Di”

	Fault
	Yellow
	Always
	Buzzer Always “Di”


-Test mode

When Device is pressed Test_Button, red LED and green LED flash alternately,then buzzer make a sound “Di-Di”.
10-Technical Specifications
	Model No.
	HS1CG-Z

	RF frequency
	868.40- 869.85 MHz 

	RF operating distance
	Up to 70m outdoor (open area) 
Up to 40m indoor

	Temperature
	-10℃~ 50℃

	Humidity
	95% max

	Weight
	148g


Net_Button





Test_Button/


LED Indicator









