SPES-02A Smart Plug

EASYSAVER Co., LTD

Slave Device : Inclusion / Exclusion

Add to the Network

- Put your Z-Wave controller into inclusion mode.

- Plug the SPES-02A into the power outlet.

- Press and hold on/off button about 5 seconds.

- Orange LED will be flashing about 2 seconds, then inclusion process will be start.

- Wait until inclusion will be done.

Remove From the Network

- Put your Z-Wave controller into exclusion mode

- Plug the SPES-02A into the power outlet.

- Press and hold on/off button about 5 seconds.

- Orange LED will be flashing about 2 seconds, then exclusion process will be start.

- Wait until exclusion will be done.

Factory Reset

Please use this procedure only when the network primary controller is missing or otherwise inoperable.

- Plug the SPES-02A into the power outlet.

- Press and hold on/off button about 10 seconds.

- Orange LED will be flashing about 2 seconds, then factory reset process will be start.

Devices from multiple manufactures in one network

SPES-02A is a wireless smartplug, based on Z-Wave Plus technology.

Z-Wave Plus enabled devices displaying the Z-Wave Plus logo can also be used with it regardless of the manufacturer,

and can also be used in other manufacturer’s Z-Wave enabled networks.

Remote On/Off control of the connected load is possible with other manufacturer’s wireless controller.

Each switch is designed to act as a repeater.

Repeaters will re-transmit the RF signal to ensure that the signal is received by its intended destination

by routing the signal around obstacles and radio dead spots.

Association Command Class

Association Command

- SPES-02A is an always listening Z-Wave device, so associations may be added or removed by a controller at any time.
Or If your controller requires to have the SPES-02A send a ‘node information frame' or NIF for associations, then press and hold the On/Off button about 5 seconds will cause the SPES-02A to send its NIF.

- There is only one group for the SPES-02A

- Association Group support is only 1. For Group 1, SPES-02A will report its latest status to Z-Wave Controller.

- Maximum 5 devices that can be added to the group

Notification

| Unsolicited Report | NOTIFICATION_REPORT | Notification Type : 0x08 Relay ON : 0x03 Relay OFF : 0x02 | The device reports current state of relay when it changes its state. (For example, when user press button or user plug in the smart plug) |  |
| --- | --- | --- | --- | --- |
|  |  | Notification Type : 0x08 Over Current : 0x06 | When over-current occurs, the device shuts off the relay. The device reports the over-current. |  |
|  |  | Notification Type : 0x08 Over Load : 0x08 | When measured watt is over 3000 Watt, the device reports this notification every 10 minute. |  |
|  |  | Notification Type : 0x08 Under Load : 0x05 | After Overload notification, the measured watt is under 2900 Watt, the device reports this notification. |  |
|  |  | Notification Type : 0x09 Hardware Fail : 0x01 | Device must report when it is not the normal operation as an internal malfunction. |  |
|  | HEAT_ALARM | Notification Type : 0x04 Over Heat : 0x02 |  | When measured heat is over 100 degree, the device reports this notification and shuts off the relay. |
|  |  | Notification Type : 0x04 Rapid Temp Rise : 0x04 |  | When measured heat is over 90 degree, the device reports this notification every 10 minute |
|  |  | Notification Type : 0x04 Under Heat : 0x06 |  | After Rapid temp rise notification, measured heat is under 80 degree, the device reports this notification. |
|  | MeterV3_REPORT | Rate Type: METER_REPORT_RATE_TYPE_IMPORT_V4(0x01) Meter Type : METER_REPORT_METER_TYPE_ELECTRIC_METER_V3(0x01) Delta Time : Transmission Period(min unit), Previous Meter Value : Previous Value |  |  |
|  |  | Scale:0x00 | Report the KWh(4byte) value. - In addition to reporting regularly even when plugged in to a power source and the first report at a time. |  |
|  | CONFIGURATION_REPORT | Parameter Number : 5 Size : 1 | When connected equipment is changed to Use in the Not Use, when the device is changed to Not Use In Use. |  |
|  |  |  | The standby power of connected devices to become this feature is enabled must be set. Or used the Default value. |  |

Configuration Command Class

| Parameter Number : 1 Size : 4 Default : 0 Possible Values : 0, 0x0003E801 | First 3 Bytes : Standby Power Setting Value Last 1 Byte : Standby Power Enable/Disable If the value is set by 1000, standby power equals 10.00w (0x0003E8) and standby power enable is 1. à total 0x0003E801 |
| --- | --- |
| Parameter Number : 2 Size : 1 Default : 1 Possible Values : 0, 1 | Value : 1 à Periodic Measurement Value Transmission Enable Value : 0 à Periodic Measurement Value Transmission disable |
| Parameter Number : 3 Size : 1 Default : 1 Possible Values : 0, 1 | Value : 0 à Power Accumulation Stop Value : 1 à Power Accumulation Start |
| Parameter Number : 4 Size : 1 Default : 0 Possible Values : 0, 1 | Value : 0 à Connected Device Not Use Value : 1 à Connected Device Use - Before setting the standby power it does not report. |
| Parameter Number : 5 Size : 1 Default : 6 Possible Values : 0 ~ 72 | Periodic Measurement Value Transmission Interval Minimum time interval : 10minutes Value : 1 (10minutes * 1) = 10minutes interval Value : 2 (10minutes * 2) = 20minutes interval …….. Value : 72(10min * 72) = 12 hours interval =>MAX |

Basic Command

| COMMAND_CLASS_BASIC | BASIC_GET | BASIC_REPORT | Report Relay Status 0x00 : Relay OFF 0xFF : Relay ON |
| --- | --- | --- | --- |
|  | BASIC_SET | 0xFF | 0xFF : Relay ON |
|  |  | 0x00 | 0x00 : Relay OFF |

When contoller send BASIC_SET with 0x00 data, SPES-02A will turn off the relay in order to cut down the AC power.

And SPES-02A will send ACK notification with POWER_MANAGEMENT type (0x08) and AC_MAINS_DISCONNECTED (0x02).

When contoller send BASIC_SET with 0xFF data, SPES-02A will turn on the relay in order to connect the AC power.

And SPES-02A will send ACK notification with POWER_MANAGEMENT type (0x08) and AC_MAINS_RECONNECTED (0x03).

These ACK notifications will be sent as well when user press Power button.

Meter Command

| COMMAND_CLASS_METER_V3 | When the device requests a METER_GET reports the value set in the Scale. | METER_REPORT |  |
| --- | --- | --- | --- |
|  |  | Rate Type: METER_REPORT_RATE_TYPE_IMPORT_V4(0x01) Meter Type : METER_REPORT_METER_TYPE_ELECTRIC_METER_V3(0x01) Delta Time : Transmission Period(in minutes), Previous Meter Value : Previous Value |  |
|  |  | Scale:0x00 | Report the KWh(4 byte) value |
|  |  | Scale:0x02 | Report the Watt(4byte) value The hex data of device must be divided by 100 because this data is 1/100 Watt unit. |
|  | METER_RESET |  | initialize the KWh value |
|  | METER_SUPPORTED_GET |  |  |
|  | METER_SUPPORTED_REPORT | Scale: 0 or 2 | Meter Type:0x01, Scale Supported:0x05 |

Security Command Class is implemented

SPES-02A is security enabled Z-Wave plus product.

Security Enabled Z-Wave Controller must be used in order to fully utilize this product.

LED indication

Relay On : GREEN LED On

Relay Off : RED LED On

Overcurrent Detected : Orange LED blinking about 60 seconds

Firmware Upgrading : Orange LED blinking until Firmware Upgrading is finished.

Inclusion / Exclusion : Orange LED blinking fastly about 3 seconds.

Factory Reset : Orange LED blinking fastly about 3 seconds.

Orange LED of Smart Plug

Button of Smart Plug