| Frequency | 908.42 MHz |
| --- | --- |
| Battery Type | CR2450 Battery 1 PCS |
| Operating Temperature | C to 55 C (14°F - 131°F) |
| Storage Temperature | C to 60 C (-4°F - 140°F) |
| Operating Air Range | Up to 152 feet line of sight |
| Low Voltage Alarm | 2.5V±0.2V |

HO-09ZW Magnetic Contact Installation Guide

Introduction

HO-09ZW  Magnetic Contact  are  designed  to  secure  the  perimeter  of  the  residential  premise,  and  provide  the ability  to  add  various  automation  services.  The  Z-Wave  Magnetic  Contact  which  consists  of  a  magnet  that

attaches  to  a  door  or  window,  will  communicate  door  events  to  the  home  security  system.  When  the  magnet  is

moved  away  from  the  sensor,  a  signal  will  be  sent  to  the  control  panel  that  communicates  the  changed  state  to  the

security  system.  Signals  can  also  be  used  to  activate  a  chime  or  convenience  lighting  based  on  system  settings.

Specification

Inclusion Sensor

For  Inclusion  a  network:  Put  the  Z-Wave  Interface  Controller  into  “Inclusion”  mode,  and following  its  instruction  to  include  HO-09ZB  to  your  controller.  This  magnetic contact  needs  to  be  included

before  installation.

1.	Pull out the insulating strip, making the device power on.  (Figure  1)

Figure 1

2.	Pressing the anti-tamper switch three times.

Figure 2

- Red LED will blink three times while inclussion the network successful  (Figure  2).

4.	If inclussion failed,  the  sensor  will  go  into  sleep  mode.  To  wake  the  sensor  again,  you

need  to  use  magnet  or  anti-tamper switch  (Figure  2)  to  trigger  an inclusion  process,  and  then  sensor  will  repeat

steps  from  3  to  4.

Exclusion  Sensor

For  Exclusion  a  network:  Put  the  Z-Wave  Interface  Controller  into  “Exclusion”  mode,

and  following  its  instruction  to  exclude magnetic contact from  your  Z-Wave  controller.

- Pressing the anti-tamper switch Triple then  enter  exclusion  mode.

- Red LED will blink three times while exclussion the network successful.

Factory  Default  Reset

Press the anti-tamper switch five times to reset.

Note:  Use  this  procedure  only  in  the  event  that  the  primary  controller  is  lost.

Installation

1.	Please  make  sure  that  the  sensor  and  magnet  is  located  less  than  14mm  from  each  other.  For  optimal

performance,  it  is  highly  recommended  to  install  the  Door/Window  sensor  on  the  fixed  frame  and  the  magnet

on  the  moving  part  of  the  door/window.  Place  the  sensor  near  the  top  of  the  door  that  close  to  the  opening

edge  of  the  door.  This  is  the  mounting  location  for  the  sensor.

2.Use  the  provided  double-sided  tape  on  the  sensor(Figure  3).  Attach  the  sensor  to  the  door.

Press  firmly  and  hold  in  placefor  a  few  seconds  .  Secure  it  with  silicone  if  needed.

Figure 3

Adhesive application surface: Acrylic, Glass

Aluminum and Stainless steel…etc.

3.	Use  provided  double-sided  tape  on  the  magnet.  Make  sure  the  alignment  of  both  sensor  and  magnet  alignment

mark  are  facing  each  other  (Figure  4).  Press  firmly  and  hold  in  place  for  a  few  seconds.  Secure  it  with

silicone  if  needed.

Figure 4

4.	HO-09ZW can  be  included   and  operated  in  any  Z-Wave  network  with  other  Z-Wave  certified  devices

from  other  manufacturers  and/or  other  applications.  All  non-battery  operated  nodes  within  the  network  will

act  as  repeaters  regardless  of  vendor  to  increase  reliability  of  the  network.

5.	Association

Support  grouping  identifier  =  1

Support  one  group  with  5  nodes.

All  triggering  report  and  low  voltage  report  will  be  sent  to  the  associated  nodes.

Z-Wave  Command  Classes

COMMAND_CLASS_ZWAVEPLUS_INFO,

COMMAND_CLASS_VERSION,
COMMAND_CLASS_MANUFACTURER_SPECIFIC,
COMMAND_CLASS_DEVICE_RESET_LOCALLY,
COMMAND_CLASS_POWERLEVEL,
COMMAND_CLASS_BATTERY,
COMMAND_CLASS_SENSOR_BINARY,
COMMAND_CLASS_NOTIFICATION，
COMMAND_CLASS_ASSOCIATION,
COMMAND_CLASS_ASSOCIATION_GRP_INFO, 
COMMAND_CLASS_WAKE_UP

|  | Switch Type | Status |
| --- | --- | --- |
| Notification Type | Reed Switch | 0x06 |
|  | Tamper Switch | 0x07 |
| Event | Reed Switch | Close:0x17, Open:0x16 |
|  | Tamper Switch | Close:0x00, Open:0x03 |
| Sensor Binary Report | Reed Switch | Close:0x00, Open:0xFF |

Notification  Type

Operation

1.	The   LED  will  stay  off  during  the  normal  operation.

2.	The  sensor  is  equipped  with  a  anti-tamper  switch.  If  the  cover  of  sensor  is  removed,  the  sensor  will  send  an  alarm

to  the  home  controller.

3.     Anti-tamper  switch  event  will  be  ignored  when  in  auto  inclusion  procedure.

This product can be included and operated in any Z-Wave network with other Z-Wave certified
devices from other manufacturers and/or other applications. All non-battery operated nodes
within the network will act as repeaters regardless of vendor to increase reliability of the network.