# Base Station

# Z-Wave                Technical Manual

# Ring Base Station

## Z-WaveTM

This product can be operated in any Z-WaveTM network with other Z-Wave certified devices from other manufacturers. All mains operated nodes within the network will act as repeaters regardless of vendor to increase reliability of the network.

SmartStart enabled products can be added into a Z-Wave network by scanning the Z-Wave QR Code present on the product with a controller providing SmartStart inclusion. No further action is required and the SmartStart product will be added automatically within 10 minutes of being switched on in the network vicinity.

## Adding Sensor to a Z-Wave Network

Ring Sensors can be added via SmartStart or via classic inclusion mode –

Ring Device Smart Start Inclusion Steps:

- Initiate add device flow in the Ring Alarm mobile application by going to Menu  Set Up a Device – Follow the guided add flow  instructions provided in the application

- Scan the QR code found on the package of the device or the QR code found on the back of the device

Non-Ring Device Smart Start Inclusion Steps:

- Initiate add device flow in the Ring Alarm mobile application by choosing Set Up a Device from the main menu.

- Choose any Ring device such as Security  Range Extenders  Range Extender (2nd Gen) -> follow on screen instructions and scan the QR code.

Classic Inclusion Steps:

- Initiate add device flow in the Ring Alarm mobile application by choosing Set Up a Device from the main menu.

- Go to Security  Works With Ring

- Select Add

- Enter learn mode on the joining device

## Removing a Sensor to a Z-Wave Network

- Ring Sensors can be removed from the network by going to Menu  Devices  Select the device you want to remove.

- Select Device Settings

- Select Remove and follow the on screen instructions to put your device into learn mode for removal.

## Access / Edit Provisioning List

- Device gets added to provisioning list after scanning QR code

- Menu  Devices  Select the device you want to access/edit.

- Complete editing and save

## Rediscovery of a Sensor’s Capabilities

- Menu  Devices  Select the device you want to reconfigure.

- Select Device Settings  Advanced Options

- Select Reconfigure Device. Note: this may set your device configurations and settings back to defaults.

## Ring Base station – Lifeline

Association

This base station has Group 1 is the “Lifeline” group, which can hold one devices. Base station will send device reset locally notification to this group.

## Ring Base Station – Reset

Factory Default Instructions

- Launch Ring app (iOS or Android)

- Navigate to Base Station Setting: Devices à Alarm Base Station à Tap on gear icon located on top right-hand corner

- Navigate to Advanced Options  Z-Wave

- Tap “Reset Z-Wave Network” and Select “Reset”

Warning: This process is irreversible. All your Z-Wave devices will be removed from your Base Station. You will have to manually reset and re-add each device to rebuild your network.

Use this procedure only in the event that the network primary controller is missing or otherwise inoperable.

Ring Base station – Learn Mode

Since Learn mode is not mandatory anymore, Learn mode is no longer supported by Ring Base station.

Ring Base station – Replication

Since Replication is not mandatory anymore, Replication is no longer supported by Ring Base station.

## Ring Base Station – Z-WaveTM Supported Command Classes

| Command Classes | Version | Required Security Classes |
| --- | --- | --- |
| Application Status | 1 | None |
| CRC16 | 1 | None |
| Inclusion Controller | 1 | None |
| Security | 1 | None |
| Security 2 | 1 | None |
| Supervision | 1 | None |
| Time | 1 | None |
| Transport Service | 2 | None |
| Z-Wave PlusTM Info | 2 | None |
| Multi Command | 1 | None |
| Indicator | 3 | Highest granted Security Class |
| Manufacturer Specific | 2 | Highest granted Security Class |
| Power Level | 1 | Highest granted Security Class |
| Version | 3 | Highest granted Security Class |
| Firmware Update | 5 | Highest granted Security Class |
| Network Management Basic | 2 | Highest granted Security Class |
| Network Management Inclusion | 3 | Highest granted Security Class |
| Network Management Proxy | 3 | Highest granted Security Class |
| Network Management Installation Maintenance | 2 | Highest granted Security Class |
| Node Provisioning | 1 | Access Control |
| Association | 2 | Highest granted Security Class |
| Association Group Info | 3 | Highest granted Security Class |
| Device Reset Locally | 1 | Highest granted Security Class |
| Multichannel Association | 3 | Highest granted Security Class |

## Ring Base Station – Z-WaveTM Controlled Command Classes

The follow section provides information on how to control minimum features of the command class in the system. In general, features are all accessible in the device’s detail page in the Ring App by going to Menu  Devices  select the device you want to control.

Basic CC

- Upon adding a binary switch device, you can go into the device details page and toggle the device ON or OFF

Door Lock CC

- Upon adding a door lock to the system, the device will be configured during the inclusion process based on it’s capabilities.

- Once configured, within the device details page, you can send commands to lock/unlock your device.

Multilevel Switch CC

- Within the devices details page, you can slide the bar to set the desired level, adjust it up or down, or stop the level change.

Sound Switch CC

- Upon adding a sound switch device such as the Ring Outdoor Siren, the volume and turning the siren on/off sound can controlled via the device details page.

Thermostat Mode and Thermostat Setpoint CC

- In the device details page, the thermostat mode can be set by toggling the on-screen buttons to Off, Heat or Cool. The temperature can be adjusted up or down as well to the desired setpoint.

Configuration CC

- When your device is included into the system, default configurations are set by the system already. If available, various configuration parameters can be modified on your device within the device’s detail page. These include things such as LED settings, brightness and volume levels, etc.