Advanced Information Product Manual

Engineering Specification

Advanced Information Product Manual

| Document No. | SPEC-KA202 |
| --- | --- |
| Version | 1.0 |
| Description | This document mainly introduces the new generation Kaadas Door Lock. The content mainly includes its interfaces, accessories, features, specifications, quick start, and software function definition. This device must be used in conjunction with a security enabled Z-Wave™ controller in order to fully utilize all implemented functions. This product can be operated in any Z-Wave™ network with other Z-Wave™ certified devices from other manufacturers. All non-battery operated nodes within the network will act as repeaters regardless of vendor to increase reliability of the network |
| Written By |  |
| Date |  |
| Reviewed By |  |
| Date |  |
| Approved By |  |
| Date |  |

| Version | Date | Brief description of changes |
| --- | --- | --- |
| 1.0 | 2023.04.29 | First revision. |

Table of Content

1	INTERFACES & ACCESSORIES	1

2	FEATURES & SPECIFICATIONS	2

2.1	Structural Characteristics	2

2.2	Hardware Characteristics	2

2.3	Software Characteristics	2

3	PRODUCT QUICK START	4

3.1	Important safety information	4

3.2	About Z-WaveTM	4

3.3	About SmartStart	4

3.4	How to add the product into Z-Wave network	4

3.5	How to remove the product from Z-Wave network	4

3.6	How to factory reset	5

3.7	Z-Wave DSK Location	5

3.8	About Product	5

3.9	About Security	5

4	SOFTWARE FUNCTION DEFINITION	6

4.1	User Behavior Interaction	6

4.2	Supported Command Classes	7

4.3	Basic Command Class mapping	7

4.4	ZWAVEPLUS_INFO	8

4.5	Manufacturer Specific	8

4.6	Version	8

4.7	Association Group Info	9

4.8	Notification	10

4.9	Door Lock	11

4.10	Door Lock Logging	11

4.11	User Code	11

4.12	Schedule	11

4.13	Battery	12

4.14	Indicator	12

4.15	Time	12

4.16	Time Parameters	12

4.17	Configuration	13

INTERFACES & ACCESSORIES

| Terminology | Description |
| --- | --- |
| Keypad | Used to operate various functions of the door lock |
| Turnpiece | For manual lock / unlock operation |
| Reset button | Restore factory settings |

FEATURES & SPECIFICATIONS

Structural Characteristics

| Parameter | Value |
| --- | --- |
| Product Identifier | KA202 |
| Color | black |
| Shell Material | kirsite |
| Shell Fire-proof Level | Frosting |
| Usage | For indoor use. |
| Operating Temperature | -25-55℃ |
| Relative Humidity | 20-80% |

Hardware Characteristics

| Parameter | Value |
| --- | --- |
| Module | EFR32ZG23A010F512GM40 |
| TX Power | Max: 14dBm |
| Antenna Distance | 40m (Indoor) /150m (Outdoor) |
| Battery Info | AA alkaline battery x4 |
| Working Current | Module: 5mA(Wake Up), 11mA（Send message） Door Lock body: 52mA(Wake Up), 190mA（operate door lock） |
| Standby Current | Module: 20uA Door Lock body: 170uA |

Software Characteristics

| Parameter | Value |
| --- | --- |
| Wireless Technology | Z-Wave™ |
| Certification Type | Z-Wave Plus™ v2 |
| Z-Wave SDK Version | 7.18.4 |
| Z-Wave Library Type | Enhanced 232 Slave |
| Z-Wave Role Type | ZWAVEPLUS_INFO_REPORT_ROLE_TYPE_SLAVE_SLEEPING_LISTENING (0x07) |
| Generic Device Type | GENERIC_TYPE_ENTRY_CONTROL (0x40) |
| Specific Device Type | SPECIFIC_TYPE_DOOR_LOCK (0x01) |
| Security Class | Non-Security, S0, S2-ACCESS |
| SmartStart | Support. After powering on, SmartStart is auto active if it’s out of the Z-Wave network. |
| Over The Air （OTA） | Support. Firmware can be updated via RF. |
| Multichannel Device | No |
| Association | Support. Refer to Section 4.7 Association Group Info. |
| Factory Reset | Support. Refer to Section 3.6 How to factory reset. |
| Power-down Memory | Support. All command settings will stay unchanged even power down. |
| Timed battery report | Support. Refer to Configuration Parameter 3. |
| Low battery warning | Support. |
| Door State Report | Support. When door lock mode changed, send out notification via Group 1. |
| User code Report | Support. When user code changed, send out notification via Group 1. |

PRODUCT QUICK START

Important safety information

Please read this Engineering Specification carefully for correct and effective use.

Failure to follow the recommendations set forth by Kaadas Limited may be dangerous or cause a violation of the law. The manufacturer, importer, distributor, and/or reseller will not be held responsible for any loss or damage resulting from not following any instruction in this guide or in other materials.

The product is intended for indoor use in dry locations only. Do not use in damp, moist, and /or wet locations.

About Z-WaveTM

Z-Wave is the international wireless protocol for communication in the Smart Home.

Z-Wave ensures a reliable communication by reconfirming every message (two-way communication) and every mains powered node can act as a repeater for other nodes (meshed network) in case the receiver is not in direct wireless range of the transmitter.

This device and every other certified Z-Wave device can be used together with any other certified Z-Wave device regardless of brand and origin as long as both are suited for the same frequency range.

If a device supports secure communication it will communicate with other devices secure as long as this device provides the same or a higher level of security. Otherwise it will automatically turn into a lower level of security to maintain backward compatibility.

About SmartStart

SmartStart enabled products can be added into a Z-Wave network by scanning the Z-Wave QR Code present on the product with a controller providing SmartStart inclusion. No further action is required and the SmartStart product will be added automatically within 10 minutes of being switched on in the network vicinity.

How to add the product into Z-Wave network

- Follow the user guide of hub to enter inclusion mode.

- Operate on lock following guide below:

- Enter master mode(Kaadas + Kaadas + Master PIN Code + Kaadas)

- Input "0" to enter "Network settings"

- Input "1 + Kaadas" to enter "Include mode"

How to remove the product from Z-Wave network

- Follow the user guide of hub to enter exclusion mode.

- Operate on lock following guide below:

- Enter master mode(Kaadas + Kaadas + Master PIN Code + Kaadas)

- Input "0" to enter "Network settings"

- Input "2 + Kaadas" to enter "Exclusion mode"

How to factory reset

- Operations on lock

- Keep the door opening and unlocking.

- Keep and hold the reset button, taking out one battery and then place it back at the same time, Keep holding the reset button for 5 seconds until you hear the sound of "BEEP".

- If reset is successful, the bolt will extend on its own to learn the orientation of the door.

2. Please use this procedure only when the network primary controller is missing or otherwise inoperable.

3. Default master pin code is “12345678”, please modify it after reset:

- Enter Master Mode(Kaadas + Kaadas + Master PIN Code + Kaadas)

- Enter "1" to select Modify Master Pin Code.

- Enter NEW 4-10 Digit Master PIN Code followed by "Kaadas"

- Repeat Step 3 to confirm NEW Master PIN Code

Z-Wave DSK Location

You can find the QR code when you open the battery box

You may also find the QR Code and DSK on individual package of each product.

Please do not remove or damage them.

About Product

This product can be operated in any Z-Wave network with other Z-Wave certified devices from other manufacturers. All mains operated nodes within the network will act as repeaters regardless of vendor to increase reliability of the network.

About Security

This device is a security enabled Z‐Wave Plus product that is able to use encrypted Z‐Wave Plus messages to communicate to other security enabled Z‐Wave Plus products. S2 Security Enabled Controller is required to operate the device.

SOFTWARE FUNCTION DEFINITION

User Behavior Interaction

| User behavior | Out of the Z-Wave network | In the Z-Wave network |
| --- | --- | --- |
| Power on | keypad light is on Send Inclusion Requests for SmartStart | keypad light is on |
| Inclusion network | Enter master mode(Kaadas + Kaadas + Master PIN Code + Kaadas) Input "0" to enter "Network settings" Input "1" + "Kaadas" to enter "Inclusion mode" | NA |
| Exclusion network | NA | Enter master mode(Kaadas + Kaadas + Master PIN Code + Kaadas) Input "0" to enter “Network settings" Input "Kaadas" + "#" to enter “Exclusion mode" |
| unlock (user code / manual / Z-Wave) | The green indicator lights up for 3 seconds | The green indicator lights up for 3 seconds Send Notification Report and Door Lock Operation Report to association group |
| Lock (Touch / Auto Lock / manual / Z-Wave) | The red indicator lights up for 3 seconds | The red indicator lights up for 3 seconds Send Notification Report and Door Lock Operation Report to association group |
| User code (Add / delete / modify) | NA | Send Notification Report and User code Report to association group |

Supported Command Classes

| Command Class | Version | Not added | Non-secure added | Securely 0 added |  | Securely 2 added |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Non-secure CC | Secure CC | Non-secure CC | Secure CC |
| ZWAVEPLUS_INFO | 2 | Support | Support | Support |  | Support |  |
| ASSOCIATION | 2 |  |  |  | Support |  | Support |
| ASSOCIATION_GRP_INFO | 3 |  |  |  | Support |  | Support |
| MULTICHANNEL ASSOCIATION | 3 |  |  |  | Support |  | Support |
| NOTIFICATION | 8 |  |  |  | Support |  | Support |
| DOOR LOCK | 4 |  |  |  | Support |  | Support |
| USER CODE | 2 |  |  |  | Support |  | Support |
| SCHEDULE | 4 |  |  |  | Support |  | Support |
| DOOR LOCK LOGGING | 1 |  |  |  | Support |  | Support |
| CONFIGURATION | 4 |  |  |  | Support |  | Support |
| APPLICATION STATUS | 1 | Support | Support | Support |  | Support |  |
| TIME | 1 | Support | Support | Support |  | Support |  |
| TIME PARAMETERS | 1 |  |  |  | Support |  | Support |
| VERSION | 3 |  |  |  | Support |  | Support |
| MANUFACTURER_SPECIFIC | 2 |  |  |  | Support |  | Support |
| DEVICE_RESET_LOCALLY | 1 |  |  |  | Support |  | Support |
| POWERLEVEL | 1 |  |  |  | Support |  | Support |
| INDICATOR | 3 |  |  |  | Support |  | Support |
| BATTERY | 1 |  |  |  | Support |  | Support |
| SECURITY | 1 | Support | Support | Support |  | Support |  |
| SECURITY_2 | 1 | Support | Support | Support |  | Support |  |
| TRANSPORT_SERVICE | 2 | Support | Support | Support |  | Support |  |
| SUPERVISION | 1 | Support | Support | Support |  | Support |  |
| FIRMWARE_UPDATE_MD | 5 |  |  |  | Support |  | Support |

Basic Command Class mapping

Basic Command maps to Door Lock Command Class, as shown below.

| Command | Value | Mapped | Value | Function |
| --- | --- | --- | --- | --- |
| Basic Set | 0x00 | Door Lock Operation Set | Door Unsecured | Open the door |
|  | 0xFF |  | Door Secured | Close the door |
| Basic Report | 0x00 | Door Lock Operation Report | Door Unsecured | The door is opened |
|  | 0xFF |  | Door Secured | The door is closed |
| Basic Get |  | Door Lock Operation Get |  |  |

ZWAVEPLUS_INFO

The Command is used to differentiate between Z-Wave Plus v2, Z-Wave for IP and Z-Wave devices. This command provides additional information about the Z-Wave Plus v2 device in question.

| Parameter | Value |
| --- | --- |
| Z-Wave Plus Version | 2 |
| Role Type | 7 (ZWAVEPLUS_INFO_REPORT_ROLE_TYPE_SLAVE_SLEEPING_LISTENING) (APPLICATION_FREQ_LISTENING_MODE_1000ms) |
| Node Type | 0 (ZWAVEPLUS_INFO_REPORT_NODE_TYPE_ZWAVEPLUS_NODE) |
| Installer Icon Type | 0x0300 (ICON_TYPE_GENERIC_DOOR_LOCK_KEYPAD) |
| User Icon Type | 0x0300 (ICON_TYPE_GENERIC_DOOR_LOCK_KEYPAD) |

Manufacturer Specific

The Command is used to advertise manufacturer specific information.

| Parameter | Value |
| --- | --- |
| Manufacturer ID 1 | 0x02 |
| Manufacturer ID 2 | 0x1D |
| Product Type ID 1 | 0x00(EU) / 0x01(US) / 0x02(AU) |
| Product Type ID 2 | 0x03 |
| Product ID 1 | 0x00 |
| Product ID 2 | 0xCA (202) |

Version

The Command may be used to obtain the Z-Wave library type, the Z-Wave protocol version used by the application, the individual command class versions used by the application.

| Parameter | Value |
| --- | --- |
| Z-Wave Protocol Library Type | 0x03 |
| Z-Wave Protocol Version | 0x07 |
| Z-Wave Protocol Sub Version | 0x12 |
| Firmware 0 Version | Z-Wave Chip Firmware Version Major |
| Firmware 0 Sub Version | Z-Wave Chip Firmware Version Minor |
| Hardware Version | 0xCA (202) |
| Number of firmware targets | 0x01 |
| Firmware 1 Version | Door Lock Firmware Version Major |
| Firmware 1 Sub Version | Door Lock Firmware Version Minor |

Association Group Info

The Command is used to manage associations to Node ID destinations.

| ID | Name | Count | Profile | Function |
| --- | --- | --- | --- | --- |

| 1 | Lifeline | 5 | General: Lifeline (0x0001) | Battery Report(0x8003)： Issued periodically to report the current battery level; Issued when battery becomes low. Device Reset Locally Notification (0x5A01) ： Issued when Factory Reset is performed. Door Lock Operation Report(0x6203)： Issued when door lock mode changed Door Lock Configuration Report(0x6206)： Issued when door lock configuration changed Indicator Report(0x8703)： Issued when indicator set received Notification Report(0x7105)： Issued when door lock event changed User Code Report(0x6303)： Issued when user code changed by keypad Master Code Report(0x6310) Issued when master code changed by keypad User Code KEYPAD MODE Report(0x630A) Issued when keypad mode changed by keypad Configuration Report(0x7006) Issued when the volume configuration value is changed by keypad |
| --- | --- | --- | --- | --- |
| 2 | On/Off control | 5 | General: Control (0x2000) | Basic Set (0x2001) ： Issued when door lock mode changed (value is determined by parameter 0x04) |

Notification

The Command is used to advertise events or states, such.

Notification Type = Access Control (0x06)，according to Table 4-8-1

Table 4-8-1:

| Description | Event | Alarm type | Alarm Level | Parameters |
| --- | --- | --- | --- | --- |
| Manual lock operation (mechanical key or thumb turn) | 0x01 | 0x15 | 0x01 | 0x01 |
| Manual unlock operation (mechanical key or thumb turn) | 0x02 | 0x16 | 0x01 | 0x01 |
| RF lock operation | 0x03 | 0x18 | 0x01 | 0x01 |
| RF unlock operation | 0x04 | 0x19 | 0x01 | 0x01 |
| Keypad lock operation | 0x05 | 0x15 | 0x02 |  |
| Keypad unlock operation | 0x06 | 0x13 | User ID | User Code Report |
|  |  |  | 0xFB |  |
| Auto lock locked operation | 0x09 | 0x1B | 0x01 |  |
| Lock jammed | 0x0B | 0x00 | 0x00 |  |
| All user codes deleted | 0x0C | 0x21 | 0xFF |  |
| Single user code deleted | 0x0D | 0x21 | User Id | User Code Report |
| New user code added (add and modify) | 0x0E | 0x70 | User Id | User Code Report |
| New user code not added due to duplicate code | 0x0F | 0x71 | User Id | User Code Report |
| New program code entered : unique code for lock configuration | 0x12 | 0x70 | 0x00 | Master Code Report |
| Manually enter user access code exceeds code limit | 0x13 | 0x00 | 0x00 |  |

Door Lock

The Command is used to operate and configure a door lock device.

Support Door Lock Mode: Door Secured (0xFF), Door Unsecured (0x00)

The auto relock time mapping table of door lock and Z-Wave is as follows:

| Z-Wave definition | Door lock function |
| --- | --- |
| Auto-relock time = 0 | disabled |
| Auto-relock time = 1 - 30 | 30 Seconds |
| Auto-relock time = 31 - 60 | 60 Seconds |
| Auto-relock time = 61 - 120 | 120 Seconds |
| Auto-relock time = 120 – 65535 | 180 Seconds |

Door Lock Logging

The Command provides an audit trail in an access control application. Each time an event takes place at the door lock, the system logs the user's ID, date, time etc.

User Code

The Command is used to manage User Codes in access control systems

- Support master code

- Supported User ID Status: available(0x00), enabled/grant access (0x01), Disable(0x02)

- Supported Keypad Modes: Normal mode, Vacation mode, Privacy mode

In Privacy mode, it will ignore Door Lock Operator Set Command, and will send Application Rejected Request.

Schedule

The Command allows scheduling the execution of commands for a given duration in a supporting device. It is a generic Command Class that may be used to schedule commands of any other Command Class.

- SCHEDULE_SUPPORTED_REPORT:

- Number of Supported Schedule IDs: 10

- Support Enable / disable: 0

- Fall-back Support: 0

- Start Time Support: 1

- Number of supported CC: 1

- Supported CC 1: USER_CODE_COMMAND_CLASS

- Supported Command1: 0x01 (Only Set Commands are supported)

- Number of Supported Schedule Blocks: 1

- SCHEDULE_SUPPORTED_COMMANDS_REPORT：

- Supported Command Class: USER_CODE_COMMAND_CLASS

- Supported Command: USER_CODE_SET

Battery

The Command is used to request and report battery levels for a given device.

1. If send Battery Get to the device, it will issue Battery Report with current battery level to the requester.

2. If re-power on, it will issue Battery Report with current battery level via Lifeline.

3. If waked up, it will detect battery level, and issue Battery Report (Value=0xFF) via Lifeline when battery level is less than 20%.

4. Support timed battery report, refer to Configuration Parameter 3.

Indicator

The Command is used to help end users to monitor the operation or condition of the application provided by a supporting node.

| Indicator ID |  | Property ID |  |
| --- | --- | --- | --- |
| Node Identify | 0x50 | On Off Period | 0x03 |
|  |  | On Off Cycles | 0x04 |
|  |  | On time within an On/Off period | 0x05 |

Time

The Command is used to read date and time from a supporting node in a Z-Wave network.

Time Parameters

The Command is used to set date and time in a device hosting this facility

Configuration

The Command allows product specific configuration parameters to be changed.

Note: No Bulk Support equals to True. It will return an Application Rejected Request Command when receiving Configuration Bulk Set or Get (if received without Supervision encapsulation). It will reset all its configuration parameters if either manually reset to factory default or receives a Configuration Default Reset Command. It will NOT modify or reset any configuration parameter when being included or excluded of a Z-Wave network.

| Parameter | 0x01 (1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Configure volume level |  |  |  |  |
| Info | Configure volume level |  |  |  |  |
| Properties | Size |  | 1 | Min Value | 0 |
|  | Format |  | Enumerated | Max Value | 2 |
|  | Read-only |  | False | Default Value | 2 |
|  | Altering capabilities |  | False | Advanced | False |
| Description | Configure volume level |  |  |  |  |
|  | Value | Function |  |  |  |
|  | 0 | silent |  |  |  |
|  | 1 | Low volume |  |  |  |
|  | 2 | High volume |  |  |  |

| Parameter | 0x02 (2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Configure led |  |  |  |  |
| Info | Enable/disable led |  |  |  |  |
| Properties | Size |  | 1 | Min Value | 0 |
|  | Format |  | Enumerated | Max Value | 1 |
|  | Read-only |  | False | Default Value | 1 |
|  | Altering capabilities |  | False | Advanced | False |
| Description | LED on the inside of the product. |  |  |  |  |
|  | Value | Function |  |  |  |
|  | 0 | OFF |  |  |  |
|  | 1 | ON (blink) |  |  |  |

| Parameter | 0x03 (3) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Timed battery report |  |  |  |  |
| Info | Configure timed battery report |  |  |  |  |
| Properties | Size |  | 2 | Min Value | 0 |
|  | Format |  | Unsigned Integer | Max Value | 14400(0x3840) (10 days) |
|  | Read-only |  | False | Default Value | 1440(0x05A0) (one day) |
|  | Altering capabilities |  | False | Advanced | False |
| Description | Enable/Disable timed battery report and sets how often battery is reported in minutes. |  |  |  |  |
|  | Value | Function |  |  |  |
|  | 0 | Disable timed battery report |  |  |  |
|  | 1-14400 | Enable timed battery report and sets how often battery is reported in minutes. |  |  |  |

| Parameter | 0x04 (4) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Association Group 2 Settings |  |  |  |  |
| Info | Configure the Basic Set value |  |  |  |  |
| Properties | Size |  | 1 | Min Value | 0 |
|  | Format |  | Enumerated | Max Value | 6 |
|  | Read-only |  | False | Default Value | 1 |
|  | Altering capabilities |  | False | Advanced | False |
| Description | Determine the Basic Set value to control other Z-Wave devices directly. |  |  |  |  |
|  | Value | Function |  |  |  |
|  | 0 | Disable completely. |  |  |  |
|  | 1 | Unlocked Status sends Basic SET 0xFF, Locked Status sends Basic SET 0x00. |  |  |  |
|  | 2 | Unlocked Status sends Basic SET 0x00, Locked Status sends Basic Set 0xFF. |  |  |  |
|  | 3 | Sends only Basic SET 0xFF when Unlocked Status happens. |  |  |  |
|  | 4 | Sends only Basic SET 0x00 when Locked Status happens. |  |  |  |
|  | 5 | Sends only Basic SET 0x00 when Unlocked Status happens. |  |  |  |
|  | 6 | Sends only Basic SET 0xFF when Locked Status happens. |  |  |  |

| Parameter | 0x05 (5) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Check fingerprint |  |  |  |  |
| Info | Check if there is any fingerprint stored in the lock |  |  |  |  |
| Properties | Size |  | 1 | Min Value | 0 |
|  | Format |  | Enumerated | Max Value | 1 |
|  | Read-only |  | True | Default Value | 0 |
|  | Altering capabilities |  | False | Advanced | False |
| Description | No fingerprint exist/Stored fingerprint |  |  |  |  |
|  | Value | Function |  |  |  |
|  | 0 | No(No fingerprint exist). |  |  |  |
|  | 1 | Yes(Stored fingerprint). |  |  |  |