# Yale Locks

Z-Wave Plus ® v2 System Integrators Guide

Yale Assure Electronic Deadbolts

YDM732-Z7

Document Revision: 1.0

July 11, 2022

Contents

Revision History	4

Yale Z-Wave Plus ® Product Info	5

Network Operations	5

Enroll/Add device to network (SmartStart)	5

Enroll/Add device to network (Classic Inclusion Mode)	5

Un-enroll/Remove device from network (Exclusion Mode)	5

Factory Reset	6

Supported Command Classes	6

Command Class Z-Wave Plus ® Info, Version 2	6

Command Class Manufacturer Specific, Version 2*	7

Command Class Security, Version 1	7

Command Class Security 2, Version 1	7

Command Class Device Reset Locally, Version 1*	7

Command Class Power Level, Version 1*	7

Command Class Version, Version 3*	8

Command Class Battery, Version 1*	9

Command Class Door Lock, Version 4*	9

Command Class Door Lock Logging, Version 1*	9

Command Class Schedule Entry Lock, Version 3*	9

Command Class User Code, Version 2*	9

Command Class Time Parameters, Version 1*	13

Command Class Time, Version 2	13

Command Class Firmware Update Meta Data, Version 5*	13

Command Class Association, Version 2*	14

Command Class Multi Channel Association, Version 3*	14

Command Class Association Group Info, Version 3*	16

Command Class Notification, Version 8*	17

Command Class Configuration, Version 4*	21

Command Class Application Status, Version 1	21

Command Class Transport Service, Version 2	21

Command Class Supervision, Version 1	21

Command Class Indicator, Version 3*	22

Command Class Basic, Version 2*	23

* This command class requires security.

# Revision History

| Rev. | Details |
| --- | --- |
| 1.0 | Initial Release |

# Yale Z-Wave Plus ® Product Info

Manufacturer ID: ASSA ABLOY (0x0129)

Z-Wave ® Device Type: Door Lock Keypad

Z-Wave ® Role Type: Listening Sleeping Slave (LSS)

# Network Operations

## Enroll/Add device to network (SmartStart)

SmartStart enabled products can be added into a Z-Wave ® network by scanning the Z-Wave ® QR Code present on the product with a controller providing SmartStart inclusion. No further action is required and the SmartStart product will be added automatically within 10 minutes of being switched on in the network vicinity.

- Open the Z-Wave ® system’s smart home app via smartphone or tablet and follow the in-app prompts to add a new device.

- SmartStart works when the Z-Wave ® system has the DSK saved and one of the following are true:

  - The lock has the minimum Lock firmware version AND is in a factory-reset state:

    - YDM732-Z7: v3.6.0

  - An internal key has already been established.

## Enroll/Add device to network (Classic Inclusion Mode)

- Enter the 4–8-digit Master PIN code followed by Depressing the Register button.

- Press the 9 key followed by the # key.

- Press the  key followed by the # key.

- Scan the QR code, if prompted, or…

- Enter the first five (5) digits of the DSK if prompted.

## Un-enroll/Remove device from network (Exclusion Mode)

- Enter the 4–8-digit Master PIN code followed by Depressing the Register button.

- Press the 9 key followed by the # key.

- Press the  key followed by the # key. Should be # not *

- Enter the 4–8-digit Master PIN code followed by # key.

When the Yale lock is unenrolled/excluded from the network through the device menu mode, any changes previously made to the user code database and configuration settings will be retained, as opposed to set back to defaults.

## Factory Reset

- Factory resetting the lock with the Z-Wave ® module installed will clear the Z-Wave ® network settings, causing the device to be removed from the network.

- Please use the manual factory reset procedure only when the network primary controller is missing or otherwise inoperable.

- The following is the method of performing a factory reset:

# Supported Command Classes

The Yale Assure Z-Wave Plus ® deadbolts follow the Z-Wave ® Command Class Specifications for all command classes that are implemented. Please refer to these specifications for specifics on how each command class works. The supported command classes are listed below, and certain sections contain details about operations that may be specific to the Yale lock. If a section is blank, then please refer to the Z-Wave ® specifications.

As a secure device, most of the command classes supported by the lock are required to be sent securely with Z-Wave ® security. During enrollment, the controller can use the Security Command Class to get this list directly from the lock. If a command class requires security, it is also indicated as such below.

Specification used: Z-Wave ® Specifications Release Dec 2021 BCD

## Command Class Z-Wave Plus ® Info, Version 2

The Z-Wave Plus ® Info command class reports the following information:

- Role Type: Slave Sleeping Listening (0x07)

- Node Type: Z-Wave Plus ® Node (0x00)

- Installer Icon Type: 0x0300

- User Icon Type: 0x0300

## Command Class Manufacturer Specific, Version 2*

* This command class requires security.

The Manufacturer Specific command class reports the following information:

- Manufacturer ID: 0x0129

  - This is the manufacturer ID assigned to ASSA ABLOY.

- Product ID:

  - The Product ID can be used to differentiate between hardware platforms, as well as between ZW2 and ZW3. See Table 1 - First 2 Digits of Product ID, below, for details.

  - Product IDs for the lock covered in this document are as follows:

    - 0x6103 for YDM732-Z7 (Keyless Touch Screen Deadbolt\Japan)

- Product Type ID:

  - 0xE005 for YDM732-Z7 (Keyless Touch Screen Deadbolt\Japan)

## Command Class Security, Version 1

This command class has been implemented per the Z-Wave ® Specification.

## Command Class Security 2, Version 1

This command class has been implemented per the Z-Wave ® Specification.

## Command Class Device Reset Locally, Version 1*

* This command class requires security.

The Yale door locks covered in this guide can be reset to their factory default settings by manually resetting the lock by following the procedure outlined in the specific lock’s manual.

Upon factory reset, all Z-Wave ® network settings are cleared, all the user codes are erased from the lock (including the master code), and all configurable settings are reset to default values, except for the language setting. A factory reset leaves the lock in a completely unsecure state (waiting for master code to be set), so care should be taken if using the configuration parameter to perform a remote reset. However, if the DUT is unenrolled/excluded from the network through the device menu mode, then the user code database and configuration settings will not be reset to the defaults.

## Command Class Power Level, Version 1*

* This command class requires security.

This command class has been implemented per the Z-Wave ® Specification.

The Power Level command class was implemented to allow controllers to set the transmit power for the door lock. This could be useful in large networks with many nodes, so that the lock can find working routes back to the controller while transmitting at a lower power. This ensures robust routes when the normal transmit power level is restored.

Currently there is no way to initiate a low power enrollment; this command class can only be used once the lock is enrolled successfully.

## Command Class Version, Version 3*

* This command class requires security.

The Yale Real Living locks are a multi-processor system with 1 additional firmware target. All processors can be updated through the Firmware Update Meta Data command class. The firmware targets are numbered as follows:

- Firmware Target 0 = Z-Wave ® Chip

- Firmware Target 1 = Lock Processor

To identify the firmware version for each target, the hex data in the firmware version report must be converted to decimal prior to combining major and minor version into the full version.

After a controller sends a Version Get command the log will display the Version Report similar to the below:

The above Version Report will be defined as this in the Z-Wave ® sniffer tool, Zniffer:

For Firmware Target 0, the Firmware 0 Version (0x02) and Sub version (0x22) translate to module firmware decimal value of “2.34”.

For Firmware Target 1 (the data under vg1), Firmware Version (0x2C) and Sub version (0x00) translate to lock firmware decimal value of “4.3.00”.

## Command Class Battery, Version 1*

* This command class requires security.

Per the Z-Wave Plus ® Specification, the lock will send a Battery Report with a value of 0xFF to the Lifeline node when a critical battery level is reached (starting at about 4.3V for Product Type ID 0xE005.

Low battery alarms will be generated if the lock is in a low battery state during one of the following events: any motor activation (keypad lock/unlock, RF lock/unlock, etc.), controller sends Get Battery command, or the unsolicited battery report was triggered. Yale locks will generate an unsolicited Battery Report every 8 hours if a node is listed in the Lifeline group.

## Command Class Door Lock, Version 4*

* This command class requires security.

Yale Z-Wave Plus ® locks support three door lock modes: Door Secured (0xFF), Door Unsecured (0x00), and Door Unsecured with timeout (0x01). When Auto Relock is enabled, the lock will automatically relock after all unlock events. Yale Z-Wave Plus ® locks do not support any of the “Door Unsecured for outside Door Handles” (0x20, 0x21) or “Door Unsecured for inside Door Handles” (0x10, 0x11) modes.

## Command Class Door Lock Logging, Version 1*

* This command class requires security.

This command class has been implemented per the Z-Wave ® Specification.

## Command Class Schedule Entry Lock, Version 3*

* This command class requires security.

Yale locks support Year Day Schedule types and Daily Repeating Schedule types. Yale locks allow the controller to apply a single schedule to a single user. Each user has 1 Year Day Schedule slot (Slot ID 1) and 1 Daily Repeating slots (Slot IDs 1). If user scheduling is used in the lock, then the controller MUST set the lock’s time using the Time Parameters command class.

## Command Class User Code, Version 2*

* This command class requires security.

Versions 1 and 2 of this command class can address user code slots 1 through 5 and 251(0xFB) via the User Code Set/Get/Report commands.

The range of valid users is from 1 to 5, the special 251(0xFB) is ONLY a workaround for changing the master code user.

Version 2 of this command class also includes extended versions of each of these commands, used to address the extended range of users.

The master code can be accessed (read/write) using slot 251 (0xFB), if using version 1 of this command class. For version 2, the Master Code Set/Get/Report commands must be used.

Yale locks do not support bulk commands (setting or getting multiple user codes at once) or CRC functionality for this command class.

The following implementation notes apply specifically to non-access user codes:

- The usage of non-access users has changed slightly with ZW3, compared to ZW2, but is still backwards compatible. If a User Code Set is transmitted using version 1 of the command class, then the lock will accept a value of 0x04 as the status for the non-access user.

- Previously, a value of 0x04 was reserved for setting Non-Access users, as stated above. When using version 2 of this command class, a Non-Access (now called “Messaging”) user ID status is assigned a value of 0x03. This value of 0x03 should be used with the Extended User Code Set command.

- Any available user code slot (except the master code) can be used to store a non-access user code.

- Schedules can be applied to non-access users.

Yale locks support the following User ID Status values:

Table 2 - User ID Status User Code CC v1 vs v2

| User ID Status | User Code CC v1 Set | User Code CC v1 Report Value |
| --- | --- | --- |
| Description | Value | Value |
| Available | 0x00 | 0x00 |
| Enabled / Grant Access | 0x01 | 0x01 |
| Disabled | 0x02 | 0x03 |
|  | 0x03 |  |
| Messaging: The user code is accepted, but the lock does not grant access to the user. Instead, it generates an alarm to the Lifeline and does NOT take preventative actions for further attempts to enter the User ID and/or User Code. | 0x04 | 0x04 |
| One-Time Use: This PIN is disabled immediately after being used for a successful unlock operation. | 0x06 | 0x06 |

| User ID Status | User Code CC v2: Extended User Code Set | User Code CC v2: Extended User Code Report Value |
| --- | --- | --- |
| Description | Value | Value |
| Available | 0x00 | 0x00 |
| Enabled / Grant Access | 0x01 | 0x01 |
| Disabled | 0x02 | 0x02 |
| Messaging: The user code is accepted, but the lock does not grant access to the user. Instead, it generates an alarm to the Lifeline and does NOT take preventative actions for further attempts to enter the User ID and/or User Code. | 0x03 | 0x03 |
| One-Time Use: This PIN is disabled immediately after being used for a successful unlock operation. | 0x06 | 0x06 |

## Command Class Time Parameters, Version 1*

* This command class requires security.

The controller must set the Time Parameters in the lock anytime the lock loses power. If the time is not set by the controller, then user codes with schedules applied to them cannot be granted access. When the lock is powered up, it will generate a Notification Report to indicate to the controller that power has been applied (Alarm V1 Type = 0x82, Alarm V1 Level = 0x00, Event Type = 0x08, Event Value = 0x01). This indicates to the controller that the lock no longer has a valid time set.

If the controller does not support either the Time CC or Time Parameters CC, then scheduled users will not have access.

## Command Class Time, Version 2

The controller must set the Time Parameters in the lock anytime the lock loses power. Even though the Time CC is not secure, the Time Set command must be issued at the same or higher security level as when the device was enrolled in order for time to be set otherwise it will be rejected by the device. If the time is not set by the controller, then user codes with schedules applied to them cannot be granted access. When the lock is powered up, it will generate a Notification Report to indicate to the controller that power has been applied (Alarm V1 Type = 0x82, Alarm V1 Level = 0x00, Event Type = 0x08, Event Value = 0x01). This indicates to the controller that the lock no longer has a valid time set.

If the controller does not support either the Time CC or Time Parameters CC, then scheduled users will not have access. A time sync should occur every 8 hours, starting with the Time CC. If there is no response within a minute, the next step is to issue a Time Parameters Get to sync time.

## Command Class Firmware Update Meta Data, Version 5*

* This command class requires security.

Yale Z-Wave Plus ® locks support over-the-air (OTA) upgrading of 2 firmware targets:

- Firmware Target 0: Z-Wave ® chip

- Firmware Target 1: The lock main processor

Firmware Target 0 is used to determine the correct Z-Wave ® processor image to download. ID 1 is always 0xA5, to signal this is an ASSA ABLOY Z-Wave ® image, and ID 2 is specific to the region, with the lower nibble being 0x0 and the upper nibble being the value in Table 3 - Region-Specific Values for Firmware ID 0. Eventually the lower nibble will be used to separate builds within a specific region, but this will also be 0 for now, since there is only a single build of firmware.

Table 3 - Region-Specific Values for Firmware ID 0 (Upper Nibble)

| Region | Value |
| --- | --- |
| ANZ | 0x1 |
| CN | 0x2 |
| EU | 0x3 |
| HK | 0x4 |
| IL | 0x5 |
| IN | 0x6 |
| JP | 0x7 |
| KR | 0x8 |
| RU | 0x9 |
| US | 0xA |

Firmware 1 target will depend on which version of the lock is in use (mapped to the Product Type ID).

For YDM723 -Z7 (Touch Screen interface), Firmware 1 ID = 0xE005

After an OTA is performed (a Firmware Update Status Report should return with successful), there is an additional step internally where we write/apply the image to the lock/module. When the image is being applied to the lock, the lock is unresponsive until completion of the apply image. Once the completion of the OTA image is applied the lock silently reboots.

The following is the time it takes for each product to complete OTA image apply phase:

- For Z-Wave ® Radio Chip, ~10 seconds

- For YDM732 -Z7 (Touch Screen interface), ~ 3 minutes

## Command Class Association, Version 2*

* This command class requires security.

This command class has been implemented per the Z-Wave ® Specification.

## Command Class Multi Channel Association, Version 3*

* This command class requires security.

This command class has been implemented per the Z-Wave ® Specification.

Yale locks support only one group, which can contain up to 5 nodes.

## Command Class Association Group Info, Version 3*

* Command Class Requires Security

Yale locks support the Lifeline association group.

Table 4 - Association Table

| Group ID | Maximum Nodes | Description | Commands |
| --- | --- | --- | --- |
| 1 | 5 | Lifeline | - Command_Class Battery - Battery_Report - Command_Class_Configuration - Configuration_Report - Command_Class_Notification - Notification_Report - Command_Class_Door_Lock - Door_Lock_Operation_Report - Door Lock Configuration Report - Command_Class_Device_Reset_Locally - Device_Reset_Locally_Notification - Command_Class_User_Code - User Code Report - User Code Keypad Mode Report - Master Code Report |

The following are the actions to trigger the reports:

Table 5 – Lifeline Report Trigger Table

| Report Command | RF Trigger | Manual Trigger |
| --- | --- | --- |
| Battery Report | Any RF Lock Operation when lock is under the battery thresholds | Any manual/keypad Lock Operation when lock is under the battery thresholds or Power Cycle Lock |
| Configuration Report | Configuration Set | Change Lock Settings via Keypad |
| Notification Report (Access Control) | Any RF Lock Operation | Manual or Keypad Unlock/Lock |
| Notification Report (Power Management) | Any RF Lock Operation when lock is under the battery thresholds | Any manual/keypad Lock Operation when lock is under the battery thresholds or Power Cycle Lock |
| Door Lock Operation Report |  | Manual or Keypad Unlock/Lock |
| Door Lock Configuration Report | Door Lock Configuration Set | Enable/Disable Auto-Relock Feature via Keypad |
| Device Reset Locally Notification |  | HW Factory Reset |
| User Code Report | Add/Delete User Code | Add/Delete User Code via Keypad |
| User Code Keypad Mode Report | User Code Keypad Mode Set | Enable/Disable Vacation Mode or Privacy Mode (refer to Installation Manual) |
| Master Code Report | Master Code Set | Update/Modify Master Code via Keypad |

## Command Class Notification, Version 8*

* This command class requires security.

Table 6 - Notification Table

| Alarm Reports | Alarm type | Alarm Level | Description | Notification Type | Event |
| --- | --- | --- | --- | --- | --- |
| Deadbolt Jammed | 0x09 | 0x01 | Deadbolt jammed while locking | 0x06 | 0x0B |
|  |  | 0x02 | Deadbolt jammed while unlocking | 0x06 | 0x0B |
| Keypad Lock | 0x12 | 0x (01 – max users) | Where Alarm level represents user slot number | 0x06 | 0x05 |
| Keypad Unlock | 0x13 | 0x(01-max users) | Where Alarm level represents user slot number (0x00 = Master Code) | 0x06 | 0X06 |
| Manual Lock | 0x15 | 0x01 | by key cylinder or inside thumb-turn | 0x06 | 0x01 |
|  |  | 0x02 | by touch function (lock and leave) | 0x06 | 0x01 |
|  |  | 0x03 | By inside button | 0x06 | 0x01 |
| Manual Unlock | 0x16 | 0x01 | By key cylinder or inside thumb turn | 0x06 | 0x02 |
| RF Operate Lock | 0x18 | 0x01 | by RF module | 0x06 | 0x03 |
| RF Operate Unlock | 0x19 | 0x01 | by RF module | 0x06 | 0X04 |
| Auto Lock Operate Locked | 0x1B | 0x01 | Auto re-lock cycle complete, locked. | 0x06 | 0x09 |
| User deleted | 0x21 | 0x(01-max users) | User was deleted. Alarm level = user slot number | 0x06 | 0X0D (single) 0X0C (all) |
| Door State | 0x23 | 0x00 | Door is open | 0x06 | 0x16 |
|  |  | 0x01 | Door is closed | 0x06 | 0x17 |
| Non-Access | 0x26 | 0x(01-max users) | A Non-Access Code was entered at the lock. Where alarm level represents user slot number | 0x06 | 0xFE |
| Daily Repeating Schedule Set/Erased | 0x60 | 0x(01-max users) | Schedule(s) has been set/erased for specified user ID | 0x06 | 0xFE |
| Year Day Schedule Set/Erased | 0x62 | 0x(01-max users) | Schedule(s) has been set/erased for specified user ID | 0x06 | 0xFE |
| All Schedule Types Enabled/Disabled | 0x65 | 0x(01-max users) | Schedule(s) has been enable/disabled for specified user ID | 0x06 | 0xFE |
| Master Code changed | 0x70 | 0x00 | Master code was changed at keypad | 0x06 | 0x12 |
|  |  | 0x00 | Master code was changed over RF | 0x06 | 0x0E |
| User added |  | 0x(01-max users) | User added. Alarm level = user slot number | 0x06 | 0X0E |
| Duplicate Pin-code error | 0x71 | 0x (01-max users) | Where Alarm level represents user slot number Alarm generated in response to add user RF cmd. This alarm is not generated when attempting to add duplicate pin at the keypad. The lock simply denies it and plays the “Denied”. Trying to duplicate the master code will result in a 0x71 0x00 alarm report. | 0x06 | 0x0F |
| Disabled user entered at keypad | 0x83 | 0x(01-max users) | A disabled user pin code was entered at the keypad | 0x06 | 0xFE |
| Valid user but outside of schedule | 0x84 | 0x(01-max users) | A valid user can be both a normal user and a Non-Access user. If a non-access user is out of schedule this alarm will be sent instead of the non-access alarm. | 0x06 | 0xFE |
| Tamper Alarm | 0xA1 | 0x01 | keypad attempts exceed code entry limit | 0x06 | 0X10 |
|  |  | 0x02 | front escutcheon removed from main | 0x06 | 0xFE |
| Battery is fully charged | 0x80 | 0x05 | After a low battery alert was observed, the lock was powered down and powered back up with full battery. | 0x08 | 0x0D |
| Door Lock needs Time set | 0x82 | 0x00 | Power to the lock was restored and the locks RTC was cleared. The controller should set the time to ensure proper logging. | 0x08 | 0x01 |

** The Yale lock supports a low battery alarm: too low to operate. This alarm is sent out as a Battery Report (with value = 0xFF) through the Battery Command Class. This is the last low battery alarm level before the product stops functioning. Starting at 4.3V

## Command Class Configuration, Version 4*

Parameter Number: 0x01

Name: Volume

Info: Set Volume Level to high (3), low (2), or silent (1).

Format: 0x00

Size: 1 Byte;

Min Value: 0x01

Max Value: 0x03

Default Value: 0x03

Parameter Number: 0x02

Name: Auto Relock

Info: Set Auto Relock feature to enable or disable.

Format: 0x01

Size: 1 Byte;

Min Value: 0x00

Max Value: 0xFF

Default Value: 0xFF

## Command Class Application Status, Version 1

This command class has been implemented per the Z-Wave ® Specification.

## Command Class Transport Service, Version 2

This command class has been implemented per the Z-Wave ® Specification.

## Command Class Supervision, Version 1

This command class has been implemented per the Z-Wave ® Specification.

## Command Class Indicator, Version 3*

The Receptacle support the Indicator Command Class, version 3 and support the Indicator ID 0x50 (Identify) and Properties ID 0x03, 0x04 and 0x05. However, the value of Properties ID 0x03 only is fixed number and only supports 0x0A. The value of Properties ID 0x05 only is fixed number and only supports 0x05.

## Command Class Basic, Version 2*

* This command class requires security.

This command class is mapped to Door Lock CC:

Table 10 – Basic Mapping Overview

| Basic Command | Door Lock Mapped Command |
| --- | --- |
| Basic Set (Value) | Door Lock Operation Set (Door Lock Mode) |
| Basic Report (Current Value = 0x00) | Door Lock Operation Report (Door Lock Mode = 0x00) |
| Basic Report (Current Value = 0xFF) | Door Lock Operation Report (Door Lock Mode > 0x00) |

The Basic Get Current Value, Basic Get Duration, and Basic Get Target Value are mapped to Door Lock Operation Get and Basic Set is directly mapped to Door Lock Operation Set where the Duration is returned as is but the Value and Target Door Lock State Value of the Basic Report use the following mapping:

Table 11 – Basic Report:: Value

| Value | Level | State | Door Lock State |
| --- | --- | --- | --- |
| 0 (0x00) | 0% | Off | Unsecure |
| 1..99 (0x01..0x63) | 1..100% | On | Secure |
| 100..253 (0x64..0xFD) | Reserved | Reserved |  |
| 254 (0xFE) | Unknown | Unknown | Unknown |
| 255 (0xFF) | 100% | On | Secure |