Z-Wave Plus Info

| Z-Wave Plus Version | 2 |
| --- | --- |
| Role Type | 5 (ZWAVEPLUS_INFO_REPORT_ROLE_TYPE_SLAVE_ALWAYS_ON) |
| Node Type | 0 (ZWAVEPLUS_INFO_REPORT_NODE_TYPE_ZWAVEPLUS_NODE) |
| Installer Icon Type | 0x1500 (ICON_TYPE_GENERIC_VALVE_OPEN_CLOSE) |
| User Icon Type | 0x1500 (ICON_TYPE_GENERIC_VALVE_OPEN_CLOSE) |

Version CC

| Parameter | Value |
| --- | --- |
| Z-Wave Protocol Library Type | 0x07 |
| Z-Wave Protocol Version | 0x12 |
| Z-Wave Protocol Sub Version | 0x08 |
| Firmware 0 Version | 0x02 Z-Wave Chip Major Firmware Version |
| Firmware 0 Sub Version | 0x01 Z-Wave Chip Minor Firmware Version |
| Hardware Version | 0x04 |
| Number of firmware targets | 0x00 |

Manufacturer Specific

| Parameter | Value |
| --- | --- |
| Manufacturer ID 1 | 0x02 |
| Manufacturer ID 2 | 0x7A |
| Product Type ID 1 | 0x01 |
| Product Type ID 2 | 0x01 |
| Product ID 1 | 0x00 |
| Product ID 2 | 0x36 |

Notification CC

| Notification Type |  | Notification Events / State |  | Description |
| --- | --- | --- | --- | --- |
| Heat Alarm | 0x04 | State idle | 0x00 | Notification value for the state variable going to idle (V5) |
|  |  | Overheat detected | 0x02 | No Location Support Event |
|  |  | Underheat detected | 0x06 | No Location Support Event |
| Water Alarm | 0x05 | State idle | 0x00 | Notification value for the state variable going to idle (V5) |
|  |  | Water leak detected | 0x02 | No Location Support Event |
| Water Valve | 0x0F | State idle | 0x00 | Notification value for the state variable going to idle (V5) |
|  |  | Valve operation status | 0x01 | Event Parameter 1 byte = - 0x00 = Valve does not let the water run through - 0x01 = Valve lets the water run through |
|  |  | Valve Jammed | 0x07 | Water Valve failed to fully complete or partially completed open or close operation |
| Power Management | 0x08 | State idle | 0x00 | / |
|  |  | AC mains disconnected | 0x02 | / |
|  |  | AC mains re-connected | 0x03 | / |
|  |  | Back-up battery is low | 0x10 | / |
|  |  | Back-up battery disconnected | 0x12 | / |

Indicator CC

| Parameter | Value |
| --- | --- |
| Indicator ID | 0x50 = (Node Identify) |
| Property ID | 0x03 = (On/Off Periods) 0x04 = (On/Off Cycles) 0x05 = (On time within an On/Off period) |

Basic CC Mapping of Water Valve

| Basic CC | Map to CC | Value |
| --- | --- | --- |
| Basic Set | Binary Switch Set | 0x00 = ON / Water Valve OPEN (Valve lets water run through) 0xFF = OFF / Water Valve CLOSE (Valve doesn t let water run through) |
| Basic Report | Binary Switch Report | 0x00 = ON / Water Valve OPEN (Valve lets water run through) 0xFF = OFF / Water Valve CLOSE (Valve doesn t let water run through) |

Association Group Info (AGI)

| Association Group | Name | Node Count | Function |
| --- | --- | --- | --- |
| 1 | Lifeline | 5 | - Device Reset Locally Notification - Basic Report - Binary Switch Report - Indicator Report - Sensor Multilevel Report Temperature Auto report based on Configuration Parameter 0x22 Setting. - Heat Alarm Notification Report (0x04) - 0x00 = State idle - 0x02 = Overheat no location support - - 0x06 = Underheat no location support - Water Alarm Notification Report (0x05) - 0x00 = State idle - 0x02 = Water leak detected no location support - Water Valve Notification Report (0x0F) - 0x01 = Valve operation 0x00 = VALVE CLOSED/OFF (Valve does not let the water run through.) 0x01 = VALVE OPEN/ON (Valve lets the water run through) |
| 2 | Water Valve | 5 | - Basic Set (By default the Configuration CC parameter 0x11 (17) Inverse Water Valve report is enabled to send out following report.) - 0x00 = Let the water run through (Based on Configuration Parameter 0x13) - 0xFF = Does t let the water run through. (Based on Configuration Parameter 0x12) |
| 3 | Leak Sensor | 5 | - Basic Set - 0x00 = IDLE / CANCEL (Based on Configuration Parameter 0x32 Setting) - 0xFF = TRIGGERED (Based on Configuration Parameter 0x31 Setting) |
| 4 | Overheat Alarm | 5 | - Basic Set - 0x00 = IDLE / CANCEL (Based on Configuration Parameter 0x27 Setting) - 0xFF = TRIGGERED (Based on Configuration Parameter 0x26 Setting) |
| 5 | Freeze Alarm | 5 | - Basic Set - 0x00 = IDLE / CANCEL (Based on Configuration Parameter 0x2C Setting) - 0xFF = TRIGGERED (Based on Configuration Parameter 0x2B Setting) |

Supported Command Classes IN NIF

| Command Class | Version | Not added | Non-secure added | Security 0 added |  | Security 2 added |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Non-secure | Secure | Non-Secure | Secure |
| ZWAVEPLUS_INFO | 2 | Support | Support | Support | --- | Support | ---- |
| SWITCH_BINARY | 2 | Support | Support | --- | Support | --- | Support |
| ASSOCIATION | 3 | Support | Support | --- | Support | --- | Support |
| MULTI_CHANNEL_ASSOCIATION | 4 | Support | Support | --- | Support | --- | Support |
| ASSOCIATION_GRP_INFO | 3 | Support | Support | --- | Support | --- | Support |
| NOTIFICATION | 8 | Support | Support | --- | Support | --- | Support |
| TRANSPORT_SERVICE | 2 | Support | Support | Support | --- | Support | --- |
| VERSION | 3 | Support | Support | --- | Support | --- | Support |
| MANUFACTURER_SPECIFIC | 2 | Support | Support | --- | Support | --- | Support |
| DEVICE_RESET_LOCALLY | 1 | Support | Support | --- | Support | --- | Support |
| INDICATOR | 3 | Support | Support | --- | Support | --- | Support |
| POWERLEVEL | 1 | Support | Support | --- | Support | --- | Support |
| SECURITY | 1 | Support | Support | Support | --- | Support | --- |
| SECURITY_2 | 1 | Support | Support | Support | --- | Support | --- |
| SUPERVISION | 1 | Support | Support | Support | --- | Support | --- |
| FIRMWARE_UPDATE_MD | 5 | Support | Support | --- | Support | --- | Support |
| SENSOR_MULTILEVEL - Temperature | 11 | Support | Support | --- | Support | --- | Support |
| CONFIGURATION | 4 | Support | Support | --- | Support | --- | Support |
| APPLICATION STATUS | 1 | Support | Support | Support | --- | Support | --- |
| BASIC COMMAND | 2 | Support | Support | Support | Support | Support | Support |
| BATTERY | 2 | Support | Support | --- | Support | --- | Support |

Configuration CC

Note: No Bulk Support equals to True. It will return an Application Rejected Request Command when receiving Configuration Bulk Set or Get (if received without Supervision encapsulation). It will reset all its configuration parameters if either manually reset to factory default or receives a Configuration Default Reset Command. It will NOT modify or reset any configuration parameter when being included or excluded of a Z-Wave network

| Water Valve - 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter No. | 0x11 (17) |  |  |  |
| Name | Inverse Water Valve Report |  |  |  |
| Info | Inverse Switch Binary Report value |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x01 (1) |
|  | Read only | False | Default Value | 0x01 (1) |
|  | Altering capabilities | False | Advanced | False |
| Description | Inverses Switch Binary Report when Water Valve = 0x00 Valve doesn t let the water run through in Notification and real physical status. |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 (0) | Disable: 0x00 = does not let water run through, 0xFF = let water run through. Based on Configuration CC setting 0x12 & 0x13 |  |  |
|  | 0x01 (1) * Default Value | Enable: 0x00 = let water run through, 0xFF = does not let water run through Based on Configuration CC setting 0x12 & 0x13 |  |  |
| Parameter No. | 0x12 (18) |  |  |  |
| Name | Association Group 2 SET Value |  |  |  |
| Info | Valve SET Value when receives 0xFF |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x02 (2) |
|  | Read only | False | Default Value | 0x01 (1) |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure Association Group 2 Basic Set value when Binary Switch Report = 0xFF *(Related to Configuration setting 0x11) |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 (0) | Disable Basic Set (Send nothing) |  |  |
|  | 0x01 (1) * Default Value | Basic ON (0xFF) |  |  |
|  | 0x02 (2) | Basic OFF (0x00) |  |  |

| Water Valve - 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter No. | 0x13 (19) |  |  |  |
| Name | Association Group 2 SET Value |  |  |  |
| Info | Valve SET Value when receives 0x00 |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x02 (2) |
|  | Read only | False | Default Value | 0x02 (2) |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure Association Group 2 Basic Set value when Binary Switch Report = 0x00 *(Related to Configuration setting 0x11) |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 (0) | Disable Basic Set (Send nothing) |  |  |
|  | 0x01 (1) | Basic ON (0xFF) |  |  |
|  | 0x02 (2) * Default Value | Basic OFF (0x00) |  |  |

| Temperature Sensor - 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter No. | 0x21 (33) |  |  |  |
| Name | Temperature Report Unit |  |  |  |
| Info | Configure reporting temperature unit |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x02 (2) |
|  | Read only | False | Default Value | 0x02 (2) |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure Temperature Unit report. The default Unit is depended on Regional Frequency Setting. (By default, US=°F and EU=°C) |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 (0) | Disable Auto Temperature Report |  |  |
|  | 0x01 (1) | Report Celsius °C unit |  |  |
|  | 0x02 (2) * Default Value | Report Fahrenheit °F unit |  |  |
| Parameter No. | 0x22 (34) |  |  |  |
| Name | Temperature Threshold Change |  |  |  |
| Info | Temperature threshold to auto report |  |  |  |
| Properties | Size | 2 Bytes | Min Value | 0x0000 for °C / 0x0100 for °F |
|  | Format | Unsigned Integer | Max Value | 0x00FF for °C / 0x01FF for °F |
|  | Read only | False | Default Value | 0x0001 = 1°C / 0x0104 = 4 °F |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure Temperature threshold changed and send auto report |  |  |  |
|  | Value | Function |  |  |
|  | 0x0000 ~ 0x00FF | Higher Byte set to x0 represents Celsius °C unit |  |  |
|  | 0x0100 ~ 0x01FF | Higher Byte set to x1 represents Fahrenheit °F unit |  |  |

| Temperature Sensor 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter No. | 0x23 (35) |  |  |  |
| Name | Temperature Report Offset |  |  |  |
| Info | Configure Temperature Report Offset |  |  |  |
| Properties | Size | 2 Bytes | Min Value | 0x0000 for °C / 0x0100 for °F |
|  | Format | Unsigned Integer | Max Value | 0x10FF for °C / 0x11FF for °F |
|  | Read only | False | Default Value | 0x0000 = 0°C / 0x0100 = 0 °F |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure Temperature Offset degree; Higher byte 0x = Positive degree and 1x= Negative degree |  |  |  |
|  | Value | Function |  |  |
|  | 0x0000 ~ 0x10FF | 0x0000 ~ 0x00FF, (0 ~ +255); Higher Byte 00 = Positive Celsius +°C e.g. 0x0002 = +2°C |  |  |
|  |  | 0x1000 ~ 0x10FF, (-0 ~ -255); Higher Byte 10 = Negative Celsius -°C e.g. 0x0002 = -2°C |  |  |
|  | 0x0100 ~ 0x11FF | 0x0100 ~ 0x01FF, (0 ~ +255); Higher Byte 01 = Positive Fahrenheit +°F e.g. 0x010A = +10°F |  |  |
|  |  | 0x1100 ~ 0x11FF, (-0 ~ -255); Higher Byte 10 = Negative Fahrenheit -°F e.g. 0x110A = -10°F |  |  |
| Parameter No. | 0x24 (36) |  |  |  |
| Name | Overheat Trigger Value |  |  |  |
| Info | Configure overheat report trigger value |  |  |  |
| Properties | Size | 2 Bytes | Min Value | 0x0000 for °C / 0x0100 for °F |
|  | Format | Unsigned Integer | Max Value | 0x00FF for °C / 0x01FF for °F |
|  | Read only | False | Default Value | 0x0028=40°C / 0x0168=104 °F |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure Overheat report trigger value. Higher byte 0x00 represents Celsius °C unit, 0x01 represents Fahrenheit °F unit |  |  |  |
|  | Value | Function |  |  |
|  | 0x0000 ~ 0x00FF | From 0°C to 255°C |  |  |
|  | 0x0100 ~ 0x01FF | From 0°F to 255°F |  |  |
| Parameter No. | 0x25 (37) |  |  |  |
| Name | Overheat Recover value |  |  |  |
| Info | Configure overheat recover report value |  |  |  |
| Properties | Size | 2 Bytes | Min Value | 0x0000 for °C / 0x0100 for °F |
|  | Format | Unsigned Integer | Max Value | 0x00FF for °C / 0x01FF for °F |
|  | Read only | False | Default Value | 0x001E = 30°C / 0x0156 = 86°F |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure Overheat Recover Value |  |  |  |
|  | Value | Function |  |  |
|  | 0x0000 ~ 0x00FF | From 0°C to 255°C |  |  |
|  | 0x0100 ~ 0x01FF | From 0°F to 255°F |  |  |

| Temperature Sensor 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter No. | 0x26 (38) |  |  |  |
| Name | Association Group 4 Overheat Trigger |  |  |  |
| Info | Overheat Trigger SET value |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x02 (2) |
|  | Read only | False | Default Value | 0x00 (0) |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure Overheat Trigger Value in Association Group 4 |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 (0) * Default Value | Disable Basic Set (Send nothing) |  |  |
|  | 0x01 (1) | Send Basic ON (0xFF) |  |  |
|  | 0x02 (2) | Send Basic OFF (0x00) |  |  |
| Parameter No. | 0x27 (39) |  |  |  |
| Name | Association Group 4 Overheat Cancel |  |  |  |
| Info | Overheat Cancellation SET value |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x02 (2) |
|  | Read only | False | Default Value | 0x00 (0) |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure Overheat Cancellation SET value |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 (0) * Default Value | Disable Basic Set (Send Nothing) |  |  |
|  | 0x01 (1) | Send Basic ON (0x0FF) |  |  |
|  | 0x02 (2) | Send Basic OFF (0x00) |  |  |
| Parameter No. | 0x28 (40) |  |  |  |
| Name | Freeze Trigger Value |  |  |  |
| Info | Configure Freeze Trigger Report value |  |  |  |
| Properties | Size | 2 Bytes | Min Value | 0x0000 for °C / 0x0100 for °F |
|  | Format | Unsigned Integer | Max Value | 0x00FF for °C / 0x01FF for °F |
|  | Read only | False | Default Value | 0x0000=0°C / 0x0120=32 °F |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure Freeze Trigger Report Value |  |  |  |
|  | Value | Function |  |  |
|  | 0x0000 ~ 0x00FF | From 0°C to 255°C |  |  |
|  | 0x0100 ~ 0x01FF | From 0°F to 255°F |  |  |

| Temperature Sensor 4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter No. | 0x29 (41) |  |  |  |
| Name | Freeze Recover Value |  |  |  |
| Info | Configure Freeze Recover Report Value |  |  |  |
| Properties | Size | 2 Bytes | Min Value | 0x0000 for °C / 0x0100 for °F |
|  | Format | Unsigned Integer | Max Value | 0x00FF for °C / 0x01FF for °F |
|  | Read only | False | Default Value | 0x0002=2°C / 0x0124=36 °F |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure Freeze Recover Report Value |  |  |  |
|  | Value | Function |  |  |
|  | 0x0000 ~ 0x00FF | From 0°C to 255°C |  |  |
|  | 0x0100 ~ 0x01FF | From 0°F to 255°F |  |  |
| Parameter No. | 0x2A (42) |  |  |  |
| Name | Freeze Detection Valve Control |  |  |  |
| Info | Configure Valve Control during freeze |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x01 (1) |
|  | Read only | False | Default Value | 0x01 (1) |
|  | Altering capabilities | False | Advanced | False |
| Description | Enable / Disable Valve Control during Freeze when Water Leak is detected etected by built-in temperature sensor. Refer to Configuration CC parameter 0x28 (41) & 0x29 (42) |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 (0) | Ignore / Allowed to control Water Valve during Freeze detection |  |  |
|  | 0x01 (1) * Default Value | Forbidden to control Water Valve during Freeze detection |  |  |
| Parameter No. | 0x2B (43) |  |  |  |
| Name | Association Group 5 Freeze Trigger |  |  |  |
| Info | Configure Freeze Trigger SET value |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x02 (2) |
|  | Read only | False | Default Value | 0x00 (0) |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure Association Group 5 Freeze Detection Trigger Basic Set value |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 * Default Value | Disable Basic Set (Send nothing) |  |  |
|  | 0x01 | Basic Set ON (0xFF) |  |  |
|  | 0x02 | Basic Set OFF (0x00) |  |  |

| Temperature Sensor 5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter No. | 0x2C (44) |  |  |  |
| Name | Association Group 5 Freeze Cancel |  |  |  |
| Info | Freeze Cancellation SET value |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x02 (2) |
|  | Read only | False | Default Value | 0x00 (0) |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure Association Group 5 Freeze Detection Cancellation Basic Set value |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 * Default Value | Disable Basic Set (Send nothing) |  |  |
|  | 0x01 | Basic Set ON (0xFF) |  |  |
|  | 0x02 | Basic Set OFF (0x00) |  |  |

| Parameter No. | 0x2D (45) |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Temperature Auto Report Time Interval Set |  |  |  |
| Info | Auto Report Time in min(s) |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x01 (1) |
|  | Format | Unsigned Integer | Max Value | 0x3C (60) |
|  | Read only | False | Default Value | 0x0f (15) |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure the minimum temperature report interval in mins |  |  |  |
|  | Value | Function |  |  |
|  | 0x01 ~ 0x3C | From 1 min to 60 mins |  |  |

| Water Leak -1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter No. | 0x31 (49) |  |  |  |
| Name | Association Group 3 Water Leak Trigger |  |  |  |
| Info | Water Leak Trigger SET value |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x02 (2) |
|  | Read only | False | Default Value | 0x01 (1) |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure Association Group 3 Water Leak Trigger Basic Set value |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 (0) | Disable Basic Set (Send nothing) |  |  |
|  | 0x01 (1) | Basic Set ON (0xFF) |  |  |
|  | 0x02 (2) * Default Value | Basic Set OFF (0x00) |  |  |
| Parameter No. | 0x32 (50) |  |  |  |
| Name | Association Group 3 Water Leak Cancel |  |  |  |
| Info | Water Leak Cancellation SET value |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x02 (2) |
|  | Read only | False | Default Value | 0x00 (0) |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure Association Group Water Leak Cancellation Basic Set value |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 (0) | Disable Basic Set (Send nothing) |  |  |
|  | 0x01 (1) | Basic Set ON (0xFF) |  |  |
|  | 0x02 (2) * Default Value | Basic Set OFF (0x00) |  |  |
| Parameter No. | 0x33 (51) |  |  |  |
| Name | Water Leak Detection Valve Control |  |  |  |
| Info | Disable / Enable Water Leak Valve Control |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x01 (1) |
|  | Read only | False | Default Value | 0x01 (1) |
|  | Altering capabilities | False | Advanced | False |
| Description | Disable / Enable Valve Control when Water Leak detected |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 (0) | Disable to control Water Valve when Water Leak is detected |  |  |
|  | 0x01 (1) * Default Value | Enable to control Water Valve when Water Leak is detected |  |  |

| Water Leak - 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter No. | 0x34 (52) |  |  |  |
| Name | Water Leak Cancellation Time |  |  |  |
| Info | Report When No Longer Trigger |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Unsigned Integer | Max Value | 0x3C (60) |
|  | Read only | False | Default Value | 0x0A (10) |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure the number of seconds after which to send the leak cancel report once no further leakage is detected |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 ~ 0x3C | 0 sec ~ 60 sec |  |  |

| User Interface |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter No. | 0x41 (65) |  |  |  |
| Name | Buzzer |  |  |  |
| Info | Enable / Disable Buzzer |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x01 (1) |
|  | Read only | False | Default Value | 0x01 (1) |
|  | Altering capabilities | False | Advanced | False |
| Description | Enable / Disable Built-in Buzzer Sound |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 (0) | Disable Buzzer |  |  |
|  | 0x01 (1) * Default Value | Enable Buzzer |  |  |
| Parameter No. | 0x42 (66) |  |  |  |
| Name | LED Brightness Level |  |  |  |
| Info | Configure LED Brightness Level |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0%) |
|  | Format | Unsigned Integer | Max Value | 0x63 (99%) |
|  | Read only | False | Default Value | 0x50 (80%) |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure Built-in LED Brightness Level |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 ~ 0x63 | 0% ~ 99% |  |  |
| Parameter No. | 0x43 (67) |  |  |  |
| Name | Touch Keylock Protection |  |  |  |
| Info | Disable / Enable Touch Keylock Protection |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x01 (1) |
|  | Read only | False | Default Value | 0x00 (0) |
|  | Altering capabilities | False | Advanced | False |
| Description | Disable / Enable Touch Keylock Protection |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 (0) * Default Value | Disable Keylock Protection |  |  |
|  | 0x01 (1) | Enable Keylock Protection |  |  |

| Notification -1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter No. | 0x51 (81) |  |  |  |
| Name | Notification – Water Valve |  |  |  |
| Info | Enable / Disable Water Valve Notification Report |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x01 (1) |
|  | Read only | False | Default Value | 0x01 (1) |
|  | Altering capabilities | False | Advanced | False |
| Description | Enable / Disable Water Valve Notification Report |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 (0) | Disable Water Valve Notification Report |  |  |
|  | 0x01 (1) * Default Value | Enable Water Valve Notification Report |  |  |
| Parameter No. | 0x52 (82) |  |  |  |
| Name | Notification - Overheat |  |  |  |
| Info | Enable / Disable Overheat Notification Report |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x01 (1) |
|  | Read only | False | Default Value | 0x01 (1) |
|  | Altering capabilities | False | Advanced | False |
| Description | Enable / Disable Overheat Notification Report |  |  |  |
|  | Value | Value |  |  |
|  | 0x00 (0) | 0x00 (0) |  |  |
|  | 0x01 (1) * Default Value | 0x01 (1) * Default Value |  |  |
| Parameter No. | 0x53 (83) |  |  |  |
| Name | Notification - Freeze |  |  |  |
| Info | Disable / Enable Freeze Notification Report |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x01 (1) |
|  | Read only | False | Default Value | 0x01 (1) |
|  | Altering capabilities | False | Advanced | False |
| Description | Enable / Disable Freeze Notification Report |  |  |  |
|  | Value | Value |  |  |
|  | 0x00 (0) | 0x00 (0) |  |  |
|  | 0x01 (1) * Default Value | 0x01 (1) * Default Value |  |  |

| Notification -2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter No. | 0x53 (84) |  |  |  |
| Name | Notification – Water Leak |  |  |  |
| Info | Enable / Disable Water Leak Notification Report |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x01 (1) |
|  | Read only | False | Default Value | 0x01 (1) |
|  | Altering capabilities | False | Advanced | False |
| Description | Enable / Disable Water Leak Notification Report |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 (0) | Disable Water Leak Notification Report |  |  |
|  | 0x01 (1) * Default Value | Enable Water Leak Notification Report |  |  |
| Parameter No. | 0x55 (85) |  |  |  |
| Name | Notification - Power |  |  |  |
| Info | Enable / Disable Power Notification Report |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x01 (1) |
|  | Read only | False | Default Value | 0x01 (1) |
|  | Altering capabilities | False | Advanced | False |
| Description | Enable / Disable Power Notification Report |  |  |  |
|  | Value | Value |  |  |
|  | 0x00 (0) | 0x00 (0) |  |  |
|  | 0x01 (1) * Default Value | 0x01 (1) * Default Value |  |  |
| Parameter No. | 0x56 (86) |  |  |  |
| Name | Notification – Backup Battery |  |  |  |
| Info | Disable / Enable Backup Battery Notification Report |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x01 (1) |
|  | Read only | False | Default Value | 0x01 (1) |
|  | Altering capabilities | False | Advanced | False |
| Description | Enable / Disable Backup Battery Notification Report |  |  |  |
|  | Value | Value |  |  |
|  | 0x00 (0) | 0x00 (0) |  |  |
|  | 0x01 (1) * Default Value | 0x01 (1) * Default Value |  |  |

| Motor - 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter No. | 0x61 (97) |  |  |  |
| Name | Disable / Enable Motor Auto Training |  |  |  |
| Info | Disable / Enable Motor Auto Training |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Bitmask | Max Value | 0x03 (3) |
|  | Read only | False | Default Value | 0x03 (3) |
|  | Altering capabilities | False | Advanced | False |
| Description | Disable / Enable Motor Auto Training |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 (0) | Disable Motor Auto Training |  |  |
|  | 0x01 (1) | Enable motor auto training when not included |  |  |
|  | 0x02 (2) * Default Value | Enable motor auto training when included |  |  |
| Parameter No. | 0x62 (98) |  |  |  |
| Name | Motor Auto Training Interval |  |  |  |
| Info | Motor Auto Training Interval |  |  |  |
| Properties | Size | 1 Bytes | Min Value | 0x01 (1) |
|  | Format | Unsigned Integer | Max Value | 0x1E (30) |
|  | Read only | False | Default Value | 0x0E (14) |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure the day interval to automatically open and close the valve once |  |  |  |
|  | Value | Function |  |  |
|  | 0x01 ~ 0x1E | Each 1 day ~ 30 days |  |  |

| Motor - 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter No. | 0x63 (98) |  |  |  |
| Name | Motor Full Open Offset |  |  |  |
| Info | Motor Full Open Offset |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x01 (1) |
|  | Format | Unsigned Integer | Max Value | 0x0A (10) |
|  | Read only | False | Default Value | 0x09 (9) |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure the motor to stop opening at X% of fully open |  |  |  |
|  | Value | Function |  |  |
|  | 0x01 ~ 0x0A | 1% ~ 10% of fully open |  |  |
| Parameter No. | 0x64 (98) |  |  |  |
| Name | Motor Full Close Offset |  |  |  |
| Info | Motor Full Close Offset |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x01 (1) |
|  | Format | Unsigned Integer | Max Value | 0x0A (10) |
|  | Read only | False | Default Value | 0x04 (4) |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure the motor to stop closing at X% of fully close |  |  |  |
|  | Value | Function |  |  |
|  | 0x01 ~ 0x0A | 1% ~ 10% of fully open |  |  |

| Backup Battery |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter No. | 0x71 (113) |  |  |  |
| Name | Battery Threshold Change |  |  |  |
| Info | Battery Threshold To Auto Report |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Unsigned Integer | Max Value | 0x64 (100) |
|  | Read only | False | Default Value | 0x0A (10) |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure to send a battery report whenever battery level X% change from last report |  |  |  |
|  | Value | Function |  |  |
|  | 0x00 ~ 0x64 | 0% ~ 100% |  |  |
| Parameter No. | 0x72 (114) |  |  |  |
| Name | Low Battery Report Level |  |  |  |
| Info | Level to trigger for low battery report |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Unsigned Integer | Max Value | 0x64 (100) |
|  | Read only | False | Default Value | 0x1E (30) |
|  | Altering capabilities | False | Advanced | False |
| Description | Configure a level for trigger the low battery report |  |  |  |
|  | Value | Value |  |  |
|  | 0x00 ~ 0x64 | 0% ~ 100% |  |  |
| Parameter No. | 0x73 (115) |  |  |  |
| Name | Disable / Enable Low Battery Auto Close Valve |  |  |  |
| Info | Disable / Enable Low Battery Auto Close Valve |  |  |  |
| Properties | Size | 1 Byte | Min Value | 0x00 (0) |
|  | Format | Enumerated | Max Value | 0x01 (1) |
|  | Read only | False | Default Value | 0x01 (1) |
|  | Altering capabilities | False | Advanced | False |
| Description | Enable / Disable Backup Battery Notification Report |  |  |  |
|  | Value | Value |  |  |
|  | 0x00 (0) | Disable Auto Close Valve when low battery is triggered |  |  |
|  | 0x01 (1) * Default Value | Enable Auto Close Valve when low battery is triggered |  |  |

SmartStart Labeling

# ZAC36 comes with PIN Code, DSK string and QR Code for SmartStart and as shown in the examples below. The real QR Code can be found on product and package.

- PIN Code with QR Code on ZAC36 s housing.

# SmartStart

Pin Code: 12345

DSK:12345-xxxxx-xxxxx-xxxxx-xxxxx-xxxxx-xxxxx-xxxxx

DSK String with QR Code on Packing, the first 5 digits is PIN code for SmartStart use.

System Event Status

| Event | Detail | LED | Buzzer |
| --- | --- | --- | --- |
| System Ready | ZAC36 is Ready to operate after power on or reset. | Green LED ON 2 seconds | 2 beep sounds |
| Standalone mode heartbeat | Standalone Heartbeat without network connection | Yellow LED blinking | --- |
| Network mode heartbeat | Mesh Network Mode Heartbeat | Green LED slow blinking | --- |
| Event Success | Finished operation and success | Green LED ON 2 seconds | Short beep x 2 |
| Event Error | Operation fail or not available | RED LED blinking 3 times | Long beep x 3 |
| Event Timeout | Operation timeout | RED LED ON | 1 Pulse sound |

The Z-Wave Button Keylock

| Event | Action / Status | Key Action | LED Status | Buzzer Status |
| --- | --- | --- | --- | --- |
| Keylock Enable | Enable Lock Key Function | Long hold with 3 beep sounds& click 3 times | Yellow LED ON 1 second Red LED ON 1 second | Long beep x 1 |
|  | Success In Mesh Network Mode | --- | Yellow LED slow blinking | --- |
|  | Success InStandaloneMode | --- | Yellow LED blinking | --- |
| Keylock Disable | Disable Lock Key Function | Long hold with 3 beep sounds& click 3 times | Yellow LED blinking | Long beep x 3 |
|  | Success In Mesh Network Mode | --- | Green LED slow blinking | --- |
|  | Success InStandaloneMode | --- | Yellow LED blinking | --- |

Operation Mode

| Operation Mode | Function | Description | Action Key |  | Operation Support |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Long | Short | Standalone | Network |
| Z-Wave Network | SmartStart | Re-power up the ZAC36 unit | --- | --- | Support | Not Support |
|  | Classic Inclusion | Add into Z-Wave Mesh Network | --- | 3 | Support | Not Support |
|  | Exclusion | Remove from Z-Wave Network | --- | 3 | Support | Support |
|  | OTA | Firmware upgrade Over The Air | --- | --- | Not Support | Support |
|  | Factory Reset | Perform Device Reset Locally | 10 | 5 | Support | Support |
| Water Valve Manual Operation | Open | Control water valve to full open | --- | 1 | Support | Support |
|  | Close | Control water valve to full close | --- | 1 | Support | Support |
|  | Pause | Pause only works during open/close operation | --- | 1 | Support | Support |
|  | Resume | Resume to previous during Pause operation | --- | 1 | Support | Support |
|  | Auto-calibration | Perform calibrate position and torque force | 5 | 5 | Support | Support |
| Water Leak Alarm | Trigger to close valve | Auto close water valve | --- | 1 | Support | Support |
|  | Alarm Cancellation | Resume to normal operation mode if no alarm triggered | 2 | --- | Support | Support |

  - Network Operation & Status

| Event | Action / Status | Key Action | LED Status | Buzzer Status |
| --- | --- | --- | --- | --- |
| SmartStart | To be ready after Power Okay Event | --- | Green LED ON 2 seconds | 2 beep sounds |
|  | Enter SmartStart and Processing | --- | Yellow LED keep blinking | Keep short beep |
|  | Success | --- | Green LED ON 1 second | Short beep x 2 |
|  | Next status | --- | Green LED slow blinking | --- |
| Manual INCLUSION | Start Manual INCLUSION | Click 3 times | Green LED ON 1 second | 1 Pulse sound |
|  | Processing | --- | Yellow LED keep blinking | Keep short beep |
|  | Success | --- | Green LED ON 1 second | Short beep x 2 |
|  | Next status | --- | Green LED slow blinking | --- |
| EXCLUSION | Start EXCLUSION | Click 3 times | Green LED ON 1 second | 1 Pulse sound |
|  | Processing | --- | Yellow LED keep blinking | Keep short beep |
|  | Success | --- | Green LED ON 1 second | Short beep x 2 |
|  | Next status | --- | Green LED slow blinking | --- |
| Firmware Upgrade (OTA) | Start Triggered by Gateway | --- | Green LED ON 1 second | 1 Pulse sound |
|  | Processing | --- | Green & RED LED blinking | Keep short beep |
|  | Success Waiting SOFT REBOOT | --- | LED OFF 10 seconds | Silence 10 seconds |
|  | Next status (Power OkayFINISHED) | --- | Green LED ON 2 seconds | 2 Pulse sound |
| Factory Reset Device Reset Locally | Start Factory Reset | Long hold with 10 beep sounds& click 5 times | Yellow LED ON 1 second | --- |
|  | Success | --- | Green LED ON 2 seconds | 2 Pulse sound |
|  | Next status Standalone Mode | --- | Yellow LED keep blinking | --- |

  - Water Valve Operation & Status

| Event | Action / Status |  | Key Action | LED | Status | Buzzer Status |
| --- | --- | --- | --- | --- | --- | --- |
| OPEN Valve | Start OPEN (Valve in closed position) |  | Short Click 1 time | Yellow LED keep blinking |  | --- |
|  | Processing |  | --- | Yellow LED keep blinking |  | Keep short beep |
|  | Success |  | --- | Green | LED ON 1 second | Short beep x 2 |
|  | Next status (In Network Mode) |  | --- | Green | LED slow blinking | --- |
|  | Next status (In Standard Mode) |  | --- | Yellow LED slow blinking |  | --- |
| CLOSE Valve | Start CLOSE (Valve in opened position) |  | Click 1 time | Green | LED ON 1 second | 1 Pulse sound |
|  | Processing |  | --- | Yellow LED keep blinking |  | Keep short beep |
|  | Success |  | --- | Green | LED ON 1 second | Short beep x 2 |
|  | Next status (In Network Mode) |  | --- | Green | LED slow blinking | --- |
|  | Next status (In Standard Mode) |  | --- | Yellow LED slow blinking |  | --- |
| PAUSE Operation | Start PAUSE Only available during Open/Close operation |  | Click 1 time | Yellow LED ON 1 second Red LED ON 1 second |  | 1 Pulse sound |
|  | Processing | In Network Mode | --- | Yellow LED slow blinking |  | --- |
|  | Next status | In Network Mode | --- | Green | LED keep blinking | --- |
|  | Processing | InStandaloneMode | --- | Yellow LED keep blinking |  | --- |
|  | Next status | In Standalone Mode | --- | Yellow LED keep blinking |  | --- |
| RESUME Operation | Start RESUME Only available during Open/Close operation |  | Click 1 time | Green | LED ON 1 second | Short beep x 3 |
|  | Next Status Return Open/Close |  | --- | Yellow/Green LED blinking |  | Keep short beep |
| Manual Calibration | Start Manual Calibration |  | Long hold with 5 beep sounds & click 5 times | --- |  | --- |
|  | Processing | Open & Close 1-2 cycles | --- | Yellow LED keep blinking |  | Keep short beep |
|  | Next status | In Network Mode | --- | Green | LED slow blinking | --- |
|  | Next status | InStandaloneMode | --- | Yellow LED keep blinking |  | --- |

  - Water Leak Alarm Operation & Status

| Event | Action / Status | Key Action | LED Status | Buzzer Status |
| --- | --- | --- | --- | --- |
| Leak Sensor Probe Triggered | Start Water Leak Alarm | --- | RED LED fast blinking | Fast beep sound |
|  | Processing Close Water Valve automatically | --- | RED LED fast blinking | Fast beep sound |
| Leak Alarm Cancellation | Alarm Cancellation | Long hold with 2 beep sounds | Green LED blinking 2 times | Short beep x 2 |
|  | Success cancellation | --- | Green LED blinking 3 times | Short beep x 3 |
|  | Next status In Network Mode | --- | Green LED slow blinking | --- |
|  | Next status InStandaloneMode | --- | Yellow LED blinking | --- |