Technical User Manual For YDM3115

Table of Contents

1 FEATURES & SPECIFICATIONS	3

1.1 Hardware Characteristics	3

1.2 Software Characteristics	3

2 PRODUCT QUICK START	5

2.1 What is Z-Wave® 	5

2.2 What is SmartStart	5

2.3 How to add (pairing) the product into Z-Wave®  network	5

2.4 How to remove the product from Z-Wave®  network	5

2.5 How to factory reset for Z-Wave® 	5

2.6 Z-Wave®  DSK Location	5

2.6.1 How to locate the DSK representation on the product	5

2.6.2 How to access the DSK representation via the UI	5

3 SOFTWARE FUNCTION DEFINITION	6

3.1 Configuration Command Class	6

3.2 Supported Command Classes	6

3.2.1 Indicator Command Class	7

3.2.2 User Code Command Class	7

3.3 Basic Command Class mapping	7

3.4 Z-Wave Plus®  Info	7

3.5 Manufacturer Specific	9

3.6 Version	9

3.7 Association Group Info	9

3.7.1 Grouping identifier	10

3.7.2 Maximum number of devices that can be added to the group	10

3.7.3 Description of how the association group is used and/or triggered by the product	10

3.7.4 Description of any mapping between groups (e.g. Root Device mirrored End Point group)	10

3.7.5 Z-Wave Plus®  v2 Device Type Specification, section ‘Documentation for Association Command Class’	10

3.8 Door Lock	11

3.9 Door Lock Logging	11

3.10 Battery	11

3.11 Devices from Multiple Manufactures	11

3.12 Implemented Notification Types and Events	11

# 1 FEATURES & SPECIFICATIONS

## 1.1 Hardware Characteristics

| Parameter | Value |
| --- | --- |
| Z-Wave® Module | ZGM130S037HGN1R |
| Z-Wave® TX Power | Max: 14dBm |
| Z-Wave® Antenna Distance | 40m (Indoor) /150m (Outdoor) |
| Indicator Light Color | White |
| Input Voltage | 6.0V – 3.6V |
| Battery Info | 6.0V – 3.6V 4Cell Battery |
| Working Current | RX: 9.8 mA TX: 13.3 mA |
| Standby Current | 0.8μA |
| ## 1.2 Software Characteristics |  |

| Parameter | Value |
| --- | --- |
| Wireless Technology | Z-Wave® |
| Certification Type | Z-Wave Plus® v2 |
| Z-Wave® SDK Version | 7.20.2 |
| Z-Wave® Library Type | Enhanced 232 Slave |
| Z-Wave® Role Type | ZWAVEPLUS_INFO_REPORT_ROLE_TYPE_SLAVE_SLEEPING_LISTENING (0x07) |
| Generic Device Type | GENERIC_TYPE_ENTRY_CONTROL (0x40) |
| Specific Device Type | SPECIFIC_TYPE_DOOR_LOCK (0x01) |
| Security Class | S0, S2-ACCESS |
| SmartStart | Support. After powering on, SmartStart is auto active if it’s out of the Z -Wave network. |
| Over The Air （OTA） | Support. Firmware can be updated via RF. |
| Multichannel Device | No |

|  | Association | Support. Refer to Section 3.2 Association Group Info. |  |  |
| --- | --- | --- | --- | --- |
|  | Factory Reset | Support. Refer to Section 2.5 How to factory reset. |  |  |
|  | Power-down Memory | Support. All command settings will stay unchanged even power down. |  |  |
|  | Timed battery report | Support. |  |  |
|  | Low battery warning | Support. |  |  |
|  | Door State Report | Support. When door lock mode changed, send out notification via Group 1. |  |  |

# 2 PRODUCT QUICK START

## 2.1 What is Z-Wave®

Z-Wave®  is the international wireless protocol for communication in the Smart Home.

Z-Wave®  ensures a reliable communication by reconfirming every message (two-way communication) and every mains powered node can act as a repeater for other nodes (meshed network) in case the receiver is not in direct wireless range of the transmitter.

This device and every other certified Z-Wave®  device can be used together with any other certified Z-Wave®  device regardless of brand and origin as long as both are suited for the same frequency range.

This door lock is a security enabled Z-Wave Plus®  TM product that is capable of using encrypted Z-Wave Plus®  messages to communicate to other security enabled Z-Wave Plus®  products. It must be used in conjunction with a S2 Security Enabled Z-Wave®  TM Controller in order to fully utilize all implemented functions.

## 2.2 What is SmartStart

SmartStart enabled products can be added into a Z-Wave®  network by scanning the Z-Wave®  QR Code present on the product with a controller providing SmartStart inclusion. No further action is required and the SmartStart product will be added automatically within 10 minutes of being switched on in the network vicinity.

## 2.3 How to add (pairing) the product into Z-Wave®  network

- Operate on lock following guide below:

  - Enter Master Code-> ‘R’ -> ‘9’ -> ‘#’ -> ‘1’ -> ‘#’

## 2.4 How to remove the product from Z-Wave®  network

- Operate on lock following guide below:

  - Enter Master Code-> ‘R’ -> ‘9’ -> ‘#’ -> ‘3’ -> ‘#’ ->  Master Code -> ‘#’

## 2.5 How to factory reset for Z-Wave®

Note: Please use this procedure only when the network primary controller is missing or otherwise inoperable.

  - Press and hold the "factory reset" key after the lock body

  - Power off the lock body for more than 5S, and plug in the battery again

  - Plug in the battery again and wait for 5S to release the "factory reset" key

## 2.6 Z-Wave®  DSK Location

### 2.6.1 How to locate the DSK representation on the product

You can find the QR code when you open the battery box

### 2.6.2 How to access the DSK representation via the UI

You may also find the QR Code and DSK on individual package of each product.

Please do not remove or damage them.

# 3 SOFTWARE FUNCTION DEFINITION

## 3.1 Configuration Command Class

Parameter Number: 0x04

Name: Wrong Code Entry Limit

Info: Adjust the limit for wrong code entries allowed by your lock

Format: 0x01

Size: 1 Byte;

Min Value: 0x03

Max Value: 0x0A

Default Value: 0x05

## 3.2 Supported Command Classes

| Command Class | Version | Required Security Class |
| --- | --- | --- |
| Association | 2 | S0 or Access Control |
| Association Group Info | 3 | S0 or Access Control |
| Basic | 2 | S0 or Access Control |
| Battery | 1 | S0 or Access Control |
| Configuration | 4 | S0 or Access Control |
| Device Reset Locally | 1 | S0 or Access Control |
| Door Lock | 4 | S0 or Access Control |
| Door Lock Logging | 1 | S0 or Access Control |
| Firmware Update Meta Data | 5 | S0 or Access Control |
| Indicator | 3 | S0 or Access Control |
| Manufacturer Specific | 2 | S0 or Access Control |
| Notification | 8 | S0 or Access Control |

| Multi-Channel Association | 3 | S0 or Access Control |
| --- | --- | --- |
| Powerlevel | 1 | S0 or Access Control |
| Schedule Entry Lock | 3 | S0 or Access Control |
| Time Parameters | 1 | S0 or Access Control |
| User Code | 2 | S0 or Access Control |
| Security 0 | 1 | None |
| Security 2 | 1 | None |
| Supervision | 1 | None |
| Transport Service | 2 | None |
| Version | 3 | S0 or Access Control |
| Z-Wave Plus® Info | 2 | None |
| Application Status | 1 | None |
| Time | 2 | None |

### 3.2.1 Indicator Command Class

The Receptacle support the Indicator Command Class, version 3 and support the Indicator ID 0x50 (Identify) and Properties ID 0x03, 0x04 and 0x05. However, the value of Properties ID 0x03 only is fixed number and only supports 0x0A. The value of Properties ID 0x05 only is fixed number and only supports 0x05.

### 3.2.2 User Code Command Class

The product supports user code length of 4-10.

## 3.3 Basic Command Class mapping

Basic Command maps to Door Lock Command Class, as shown below.

| Command | Value | Mapped | Value | Function |
| --- | --- | --- | --- | --- |
| Basic Set | 0x00 | Door Lock Operation Set | Door Unsecured | Open the door |
|  | 0xFF |  | Door Secured | Close the door |
| Basic Report | 0x00 | Door Lock Operation Report | Door Unsecured | The door is opened |
|  | 0xFF |  | Door Secured | The door is closed |
| Basic Get |  | Door Lock Operation Get |  |  |

## 3.4 Z-Wave Plus®  Info

The Command is used to differentiate between Z-Wave Plus® , Z-Wave®  for IP and Z-Wave®  devices. This command provides additional information about the Z-Wave Plus®  device in question.

Parameter	Value

| Z-Wave Plus® Version | 2 |
| --- | --- |
| Role Type | 7 |
|  | (ZWAVEPLUS_INFO_REPORT_ROLE_TYPE_SLAVE_SLEEPING_LISTENING) |
|  | (APPLICATION_FREQ_LISTENING_MODE_1000ms) |
| Node Type | 0 (ZWAVEPLUS_INFO_REPORT_NODE_TYPE_ZWAVEPLUS_NODE) |
| Installer Icon Type | 0x0300 (ICON_TYPE_GENERIC_DOOR_LOCK_KEYPAD) |
| User Icon Type | 0x0300 (ICON_TYPE_GENERIC_DOOR_LOCK_KEYPAD) |

## 3.5 Manufacturer Specific

The Command is used to advertise manufacturer specific information.

| Parameter | Value |
| --- | --- |
| Manufacturer ID | 0x0129 |
| Product Type ID | 0xE017 |
| Product ID | 0x6206 |

## 3.6 Version

The Command may be used to obtain the Z-Wave®  library type, the Z-Wave®  protocol version used by the application, the individual command class versions used by the application.

| Parameter |  | Value |
| --- | --- | --- |
| Z-Wave® Protocol Library Type |  | 0x03 |
| Z-Wave® Protocol Version |  | 0x07 |
| Z-Wave® Protocol Sub Version |  | 0x14 |
| Firmware 0 | Version | 0x03 |
| Firmware 0 | Sub Version | 0x06 |
| Firmware 1 Version |  | 0x0B |
| Firmware 1 Sub Version |  | 0x02 |
| Hardware Version |  | 0x02 |
| Number of additional firmware targets |  | 0x01 |

## 3.7 Association Group Info

The Command is used to manage associations to Node ID destinations.

| ID |  | Name | Count |  | Profile |  | Function |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Device Reset Locally Notification (0x5A01) ： |  |
|  |  |  |  |  |  |  | Issued when Factory Reset is performed. |  |
|  |  |  |  |  |  |  | Battery Report(0x8003)： |  |
|  |  |  |  |  |  |  | Issued periodically to report the current battery level; |  |
|  |  |  |  |  | General: Lifeline |  | Issued when battery becomes low. |  |
| 1 |  | Lifeline | 5 |  |  |  |  |  |
|  |  |  |  |  | (0x0001) |  | Door Lock Operation Report (0x6206)： |  |
|  |  |  |  |  |  |  | Issued when door lock mode changed. |  |
|  |  |  |  |  |  |  | Indicator Report (0x8703)： |  |
|  |  |  |  |  |  |  | Triggered when green light changes state. |  |
|  |  |  |  |  |  |  | Door Lock Configuration Report (0x6206)： |  |
|  |  |  |  |  |  |  | Triggered upon a change in door lock configuration. User Code Report(0x6303): Issued when user code set Extended User Code Report(0x630D): Issued when extended user code set User Code Keypad Mode Report(0x630A): Issued when supported keypad mode set Master Code Report(0x6310): Issued when master code set Configuration Report(0x7006): Issued when configuration set Notification Report(0x7105): Issued when notification event triggered |  |

### 3.7.1 Grouping identifier

1

### 3.7.2 Maximum number of devices that can be added to the group

5

### 3.7.3 Description of how the association group is used and/or triggered by the product

### 3.7.4 Description of any mapping between groups (e.g. Root Device mirrored End Point group)

### 3.7.5 Z-Wave Plus®  v2 Device Type Specification, section ‘Documentation for Association Command Class’

## 3.8 Door Lock

The Command is used to operate and configure a door lock device.

Support Door Lock Mode: Door Secured(0xFF), Door Unsecured(0x00)

## 3.9 Door Lock Logging

The Command provides an audit trail in an access control application. Each time an event takes place at the door lock, the system logs the user's ID, date, time etc.

Max records stored = 20

## 3.10 Battery

The Command is used to request and report battery levels for a given device.

- If send Battery Get to the device, it will issue Battery Report with current battery level to the requester.

- If re-power on, it will issue Battery Report with current battery level via Lifeline.

- If waked up, it will detect battery level, and issue Battery Report (Value=0xFF) via Lifeline when battery level is less than or equal to threshold.

- Support timed battery report and it will send message every 5~10 minutes.

## 3.11 Devices from Multiple Manufactures

This product can be operated in any Z-Wave®  network with other Z-Wave®  certified devices from other manufactures. All mains operated nodes within the network will act as repeaters regardless of vendor to increase reliability of the network.

## 3.12 Implemented Notification Types and Events

Types: 0x06 - Access Control

Implemented Events:

0x01 - Manual lock operation

0x02 - Manual unlock operation

0x03 - RF lock operation

0x04 - RF unlock operation

0x06 - Keypad unlock operation

0x09 - Auto lock locked operation

0x0B - Lock jammed

0x0C - All user codes deleted

0x0D - Single user code deleted

0x0E - New user code added (The length of user code is between 4 and 8 bytes)

0x0F - New user code not added due to duplicate code

0x10 - Keypad temporary disabled

0x12 - New program code entered: unique code for lock configuration

0x13 - Manually enter user access code exceeds code limit

0x16 - Window/door is open

0x17 - Window/door is closed

Types: 0x08 - Power Management

Implemented Events:

0x01 - Power has been applied

0x0A - Replace battery soon

0x0B - Replace battery now

0x0D - Battery is fully charged